Communication protocols

SPI/I2C/UART/CAN/LIN/FLEXRAY/MQTT 21-06-2026 • Aravinth K

SPI protocol

- What is SPI?

Serial Peripheral Interface (SPI) is a synchronous, full-duplex, master-slave serial communication protocol developed by Motorola. It operates using 4 wires and is extremely common in embedded systems for sensors, flash memory, ADCs, DACs, display controllers, sim racing hardware, etc.

The core idea is simple — master and slave each have an 8-bit shift register, and they swap their shift registers bit by bit over the clock. That's it. Everything else builds on top of this.

- The 4 Lines

| Signal | Full Name | Direction | Purpose |
| --- | --- | --- | --- |
| SCLK | Serial Clock | Master → Slave | Clock generated by master only |
| MOSI | Master Out Slave In | Master → Slave | Master writes, slave reads |
| MISO | Master In Slave Out | Slave → Master | Slave writes, master reads |
| CS/SS | Chip Select | Master → Slave | Active-LOW, selects the target slave |

Important ownership rule:

- MOSI line is owned by master — master writes on it, slave reads from it

- MISO line is owned by slave — slave writes on it, master reads from it

- They never write on each other's line — that's what makes full-duplex possible without conflict

Some datasheets use SDI/SDO, DI/DO, SI/SO. SDO of master = MOSI. SDO of slave = MISO. Don't get confused by naming.

- How a Full SPI Transaction Flows (Step by Step)

This is the complete picture from start to finish:

Step 1 — CS goes LOW

Master pulls CS LOW to select the slave. This is not just a signal — this is the trigger for the first data write. The moment CS goes LOW:

- Master immediately places bit 7 (MSB) of its data on MOSI

- Slave immediately places bit 7 (MSB) of its data on MISO

Both lines are now stable with data sitting on them, waiting

Step 2 — Clock starts, first rising edge

The clock was idle (LOW for CPOL=0). Now it goes HIGH — first rising edge arrives. At this moment:

- Master reads bit 7 from MISO

- Slave reads bit 7 from MOSI

- Both are reading simultaneously — nobody is writing at this moment

- The data was already there from Step 1, so there's no conflict

Step 3 — First falling edge

Clock goes LOW. Now:

- Master writes bit 6 onto MOSI

- Slave writes bit 6 onto MISO

- Both lines change to the next bit

Step 4 — Second rising edge

Clock goes HIGH again:

- Master reads bit 6 from MISO

- Slave reads bit 6 from MOSI

Step 5 — This repeats for bits 5, 4, 3, 2, 1, 0

Step 6 — CS goes HIGH

- Transaction ends. Slave releases MISO (tri-states it). Internal state machines in the slave may latch the received command or reset their byte counter.

- The Shift Register Mental Model

Think of it this way — master has an 8-bit register, slave has an 8-bit register. They are connected in a ring:

Master SR: [b7][b6][b5][b4][b3][b2][b1][b0] ↓ MOSI ↑ MISO Slave SR: [b7][b6][b5][b4][b3][b2][b1][b0]

Every clock pulse shifts one bit from master → slave via MOSI, and one bit from slave → master via MISO at the same time. After 8 clocks they have completely swapped their registers. This is why SPI is inherently full-duplex — the swap is always happening whether you want it or not.

- Sample vs Shift — The Most Important Concept

SPI has two actions happening every clock cycle:

| Action | Meaning |
| --- | --- |
| Sample | Read the current bit from the line into your shift register |
| Shift | Push the next bit OUT onto the line |

These two actions always happen on opposite edges — always, no exceptions.

The reason is physical. If you write a new bit and read it on the same edge, you're reading the line at the exact moment it's changing — that's a setup time violation. You'd get garbage. So the protocol separates them:

- Write on one edge → line changes → stabilizes during the half-period

- Read on the next edge → data is rock stable → clean capture

- SPI Modes — CPOL and CPHA

SPI has 4 modes defined by two parameters:

CPOL (Clock Polarity) — what is the idle state of the clock?

- CPOL=0 → clock idles LOW

- CPOL=1 → clock idles HIGH

CPHA (Clock Phase) — which edge do you sample on?

- CPHA=0 → sample on leading edge, shift on trailing edge

- CPHA=1 → shift on leading edge, sample on trailing edge

| Mode | CPOL | CPHA | Clock Idle | Sample On | Shift On |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | LOW | Rising ↑ | Falling ↓ |
| 1 | 0 | 1 | LOW | Falling ↓ | Rising ↑ |
| 2 | 1 | 0 | HIGH | Falling ↓ | Rising ↑ |
| 3 | 1 | 1 | HIGH | Rising ↑ | Falling ↓ |

Mode 0 and Mode 3 are most common in real devices.

Trap: "Leading edge" is NOT always rising. If CPOL=1, clock idles HIGH, so the first transition is a falling edge — that's the leading edge. Leading = first edge after CS asserts.

- CPHA=0 Deep Dive — The Pre-load Mechanism

This is where most engineers get confused, so let's walk through it carefully.

The question that trips people: "On the first rising edge both master and slave are reading — but who wrote the data? Nobody has written yet!"

The answer: CS falling edge triggers the first write — not the clock.

The moment CS goes LOW, before the clock even starts:

- Master pre-loads bit 7 on MOSI

- Slave pre-loads bit 7 on MISO

- Data is sitting stable on both lines

Then the clock starts. The first rising edge finds data already there and simply reads it. There is no chicken-and-egg problem.

CS: ‾‾‾‾‾¦_________________________________¦‾‾‾ ↓ Master writes bit7 → MOSI Slave writes bit7 → MISO (data pre-loaded, lines stable) SCLK: ______¦‾‾‾‾‾¦_____¦‾‾‾‾‾¦_____¦‾‾ ↑ ↓ ↑ ↓ ↑ READ WRITE READ WRITE READ

So the complete CPHA=0 rule is:

| Moment | MOSI | MISO |
| --- | --- | --- |
| CS falls | Master writes bit 7 | Slave writes bit 7 |
| Rising ↑ | Slave reads MOSI | Master reads MISO |
| Falling ↓ | Master writes next bit | Slave writes next bit |
| Rising ↑ | Slave reads MOSI | Master reads MISO |

- CPHA=1 — Shift First, Sample Second

In CPHA=1 there is no pre-load on CS. Instead:

- CS goes LOW → lines idle, nothing written yet

- First clock edge (rising for CPOL=0) → both write (shift out bit 7)

- Second clock edge (falling) → both read (sample bit 7)

- Third edge → write bit 6

- Fourth edge → read bit 6

- ...and so on

The first edge is used for shifting, not sampling. This gives the data extra setup time between the write and the read — useful for slower slaves.

- Full-Duplex Clarity

When you say "both MOSI and MISO read from each other on rising edge" — yes, but with one important precision:

- Master reads MISO (slave's line) on rising edge

- Slave reads MOSI (master's line) on rising edge

- They are reading different lines simultaneously

- Nobody is writing on either line at that moment

When you say "both write on falling edge":

- Master writes new bit onto MOSI

- Slave writes new bit onto MISO

- They are writing different lines simultaneously

- Nobody is reading at that moment

This separation is what makes full-duplex clean and collision-free.

10. CS Behavior and Why It Matters

CS is not just an enable pin — it is part of the protocol state machine.

- Active LOW — asserted = 0, de-asserted = 1

- Each slave has its own CS line from master

- CS assertion (going LOW) → starts transaction, triggers pre-load in CPHA=0

- CS de-assertion (going HIGH) → ends transaction, slave latches data, resets internal byte counter, tri-states MISO

Pitfall:

Many devices require CS to toggle between bytes to reset their internal pointer. If you keep CS LOW and send multiple bytes, the slave may treat byte 2 as continuation of byte 1's command — wrong behavior.

Never assert two CS lines simultaneously. Two slaves will both drive MISO — bus contention, data corruption, possible hardware damage.

Pitfall bare metal:

Always wait for BSY flag to clear before de-asserting CS. De-asserting while BSY=1 cuts the transaction mid-way.

11. Key Rules Summary

Clock always generated by master CS active LOW — assertion triggers pre-load (CPHA=0) Sample and shift always on opposite edges PHA=0 → pre-load on CS, sample on first clock edge PHA=1 → no pre-load, shift on first clock edge Full-duplex = master and slave swap shift registers simultaneously MOSI owned by master, MISO owned by slave — no line conflict Dummy bytes needed for RX-only transactions (clock needs TX to run) Wait BSY=0 before CS de-assert in bare metal DMA buffer must be cache-coherent on Cortex-M7 Struct packing critical for SPI frame layout in C

12. One-Line Mental Models to Carry Into Interviews

- "SPI is two shift registers connected in a ring — they swap contents over 8 clocks."

- "Falling edge = write (prepare). Rising edge = read (capture). That's CPHA=0."

- "CS falling edge pre-loads bit 7 before the clock even starts — that's why the first rising edge can read without anyone writing at that moment."

- "MOSI is master's pen, MISO is slave's pen. They write on their own line, read the other's line — simultaneously, no conflict."

SPI SUMMARY cheat sheet

What is SPI?

Synchronous, full-duplex, master-slave protocol. Master and slave each have an 8-bit shift register — they swap contents over 8 clock pulses. That's the entire protocol at its core.

The 4 Lines

| Signal | Full Name | Direction | Purpose |
| --- | --- | --- | --- |
| SCLK | Serial Clock | Master → Slave | Clock generated by master only |
| MOSI | Master Out Slave In | Master → Slave | Master writes, slave reads |
| MISO | Master In Slave Out | Slave → Master | Slave writes, master reads |
| CS/SS | Chip Select | Master → Slave | Active-LOW, selects the target slave |

MOSI is master's line. MISO is slave's line. They never write on each other's line — that's full-duplex.

Transaction Flow

1. CS goes LOW → master writes bit7 on MOSI, slave writes bit7 on MISO (pre-load) 2. Rising edge → master reads MISO, slave reads MOSI (both sample simultaneously) 3. Falling edge → master writes next bit on MOSI, slave writes next bit on MISO 4. Repeat steps 2-3 for bits 6 down to 0 5. CS goes HIGH → transaction ends, slave tri-states MISO

CS falling edge pre-loads bit 7 before clock starts. First rising edge just reads — data is already there.

SPI Modes — CPOL & CPHA

| Mode | CPOL | CPHA | Clock Idle | Sample On | Shift On |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | LOW | Rising ↑ | Falling ↓ |
| 1 | 0 | 1 | LOW | Falling ↓ | Rising ↑ |
| 2 | 1 | 0 | HIGH | Falling ↓ | Rising ↑ |
| 3 | 1 | 1 | HIGH | Rising ↑ | Falling ↓ |

CPOL = idle state of clock. CPHA = which edge comes first — sample or shift.

- CPHA=0 → pre-load on CS, sample on first edge

- CPHA=1 → shift on first edge, sample on second edge

- Sample and shift are always on opposite edges — always

- Mode 0 and Mode 3 most common in real devices

Sample vs Shift — The Core Rule

Falling edge → WRITE new bit onto line (shift) Rising edge → READ bit from line (sample) ← CPHA=0 Rising edge → WRITE new bit onto line (shift) Falling edge → READ bit from line (sample) ← CPHA=1

Write on one edge → line stabilizes → read on next edge. Never same edge — that would violate setup time.

Multi-Slave Configurations

Independent CS — each slave has own CS pin, MOSI/MISO shared on bus. Master selects one at a time.

Daisy Chain — one CS for all slaves, MISO of slave N feeds MOSI of slave N+1. All slaves form one long shift register. Clock N×8 bits total. First slave gets last byte sent — plan byte order carefully.

CS Behavior — Critical Rules

Active LOW always

CS assertion → starts transaction + pre-loads bit 7 (CPHA=0)

CS de-assertion → slave latches data, resets internal byte counter, releases MISO

Many devices require CS to toggle between bytes — keeping CS LOW = slave misinterprets byte 2 as continuation

Never assert two CS lines simultaneously → MISO bus contention → data corruption

Debug Symptom → Root Cause

| Symptom | Most Likely Cause |
| --- | --- |
| Garbage data | CPOL/CPHA mode mismatch or MSB/LSB bit order wrong |
| First byte wrong, rest OK | CS→SCLK setup time violated, add delay after CS assert |
| Intermittent failures | Setup/hold violation — reduce clock speed |
| SPI hangs, BSY never clears | CS de-asserted mid-transfer or OVR not cleared |
| Works on bench, fails production | PCB trace capacitance — reduce clock speed |
| RX always 0xFF or 0x00 | MISO/MOSI swapped on board |
| DMA correct count but wrong data | Cache not flushed/invalidated (Cortex-M7) |

Q1: How does full-duplex work at hardware level?

Master and slave each have a shift register connected in a ring via MOSI and MISO. Every clock edge shifts one bit each direction simultaneously — after 8 clocks they've completely swapped registers.

Q2: How do you do RX-only SPI?

Send dummy bytes (0xFF/0x00) on MOSI to generate clock. Capture MISO. Slave ignores dummy data.

Q3: Why wait BSY before de-asserting CS?

TXE goes high when DR is empty but the byte is still shifting out in the shift register. BSY stays HIGH until the last bit is shifted. De-asserting CS while BSY=1 cuts the transfer mid-byte.

Q4: Mode 0 vs Mode 3 — practical difference?

Both sample on rising edge. Only difference is clock idle level. Most devices work on either. Matters at CS assertion moment — some devices are sensitive to the clock level when CS asserts.

Q5: Why does CPHA=0 first rising edge have data ready to read without anyone writing at that moment?

CS falling edge triggers the pre-load — master puts bit 7 on MOSI and slave puts bit 7 on MISO before the clock even starts. First rising edge finds data already stable and simply reads it.

I2C protocol cheat sheet / summary

What is I2C?

Synchronous, half-duplex, multi-master, multi-slave protocol on just 2 wires. Every slave has a unique 7-bit address baked in. Master announces the address on the bus — only matching slave responds. Everything flows on one shared bidirectional line (SDA) clocked by SCL.

The 2 Lines

| Signal | Direction | Purpose |
| --- | --- | --- |
| SCL | Master → Slave | Clock generated by master |
| SDA | Bidirectional | Address, data, ACK — everything |

Both lines are open-drain with pull-up resistors — nobody actively drives HIGH, resistor pulls HIGH passively. Any device can pull LOW anytime without short circuit. Dominant state = LOW.

VCC ¦ [Rp] ← 4.7kΩ @ 100kHz / 2.2kΩ @ 400kHz ¦ SDA ──── Master ──── Slave1 ──── Slave2

Speed Modes

| Mode | Speed |
| --- | --- |
| Standard | 100 kHz |
| Fast Mode | 400 kHz |
| Fast Mode Plus | 1 MHz |
| High Speed | 3.4 MHz |

100 kHz and 400 kHz cover 95% of real designs.

START and STOP — The Bus Language

Rule: SDA only changes when SCL is LOW during data. SDA changing while SCL is HIGH = START or STOP only — never data.

START — SDA falls while SCL is HIGH

SCL: ‾‾‾‾‾‾‾‾¦________ SDA: ‾‾‾‾¦____________ ↑ START

Bus was idle (both HIGH). Master pulls SDA LOW while SCL still HIGH → claims the bus.

STOP — SDA rises while SCL is HIGH

SCL: ________¦‾‾‾‾‾‾‾‾ SDA: ____________¦‾‾‾‾ ↑ STOP

Master releases SCL HIGH then releases SDA HIGH → bus returns to idle.

Repeated START (Sr)

New START without STOP first. Holds bus ownership while switching direction. Used for write-then-read in same transaction.

How Each Bit Flows

SCL LOW → SDA changes to new bit value (shift) SCL HIGH → SDA locked, everyone reads it (sample)

SCL: ___¦‾‾‾‾‾¦___¦‾‾‾‾‾¦___¦‾‾‾‾‾¦___ SDA: [change]‾[stable]‾[change]‾[stable] ↑ ↑ READ READ

Identical concept to SPI — change on one edge, read on other. In I2C: change when SCL LOW, read when SCL HIGH.

Address Byte — Always First After START

[A6][A5][A4][A3][A2][A1][A0][R/W] ←———————— 7-bit address ————————→ 0=Write / 1=Read

- All slaves hear this — only address-match responds

- 7-bit address in datasheet → shift left 1, OR with R/W bit for wire byte

- 0x68 address → 0xD0 on wire for write, 0xD1 for read

ACK / NACK — The Handshake After Every Byte

9th clock pulse after every 8 bits:

| State | SDA on 9th clock | Meaning |
| --- | --- | --- |
| ACK | Pulled LOW by receiver | "Got it, continue" |
| NACK | Stays HIGH (nobody pulls) | "Stop / error / done" |

- Address sent → slave ACKs if present, NACK if not there

- Master writing → slave ACKs each byte

- Master reading → master ACKs each byte except last → last byte gets NACK → tells slave to stop

Complete Transaction Flows

Write (master writes data to slave register):

S → [ADDR+W] → ACK → [REG_ADDR] → ACK → [DATA] → ACK → P

Read (master reads slave register — needs Repeated START):

S → [ADDR+W] → ACK → [REG_ADDR] → ACK → Sr → [ADDR+R] → ACK → [DATA] → NACK → P

Read multiple bytes:

S → [ADDR+W] → ACK → [REG] → ACK → Sr → [ADDR+R] → ACK → [D0] → ACK → [D1] → ACK → [D2] → NACK → P

Master ACKs every byte except the last. Last byte = NACK → STOP.

Why Repeated START — Not STOP then START?

- Between START and STOP — bus is owned by this master, nobody else can claim it

- STOP then START → another master could grab bus in between

- Also, slave's internal register pointer may reset on STOP — repeated START keeps pointer at correct register

- Write reg address → Sr → read data is atomic. Write → STOP → START → read is not.

Clock Stretching

Slave holds SCL LOW to pause transaction when it needs more processing time.

SCL: __¦‾‾‾¦______________¦‾‾‾¦__ ↑ Slave holds LOW Master waits... Slave releases → resumes

Only possible because open-drain — slave can pull SCL LOW anytime. SPI has no equivalent — slave has zero clock control.

Pitfall: Some master’s don't support clock stretching. Slave stretches + master ignores it = corrupted transaction. Always verify master controllers support it.

Multi-Master Arbitration

Two masters START simultaneously → arbitration resolves without corruption:

- Both drive SDA, both monitor SDA simultaneously

- First master to drive HIGH while other drives LOW → reads LOW back → loses → backs off immediately

- Winner never knows contest happened — transaction completes perfectly

- Works because open-drain = wired-AND — LOW always wins

I2C Address Space

| Range | Status |
| --- | --- |
| 0x00 | General call (broadcast all slaves) |
| 0x01–0x07 | Reserved |
| 0x08–0x77 | Valid 7-bit addresses (112 usable) |
| 0x78–0x7F | Reserved for 10-bit addressing |

Debug Symptom → Root Cause

| Symptom | Most Likely Cause |
| --- | --- |
| No ACK after address | Wrong address — 7-bit vs 8-bit confusion |
| No ACK, address correct | Slave not powered or pull-ups missing |
| Works 100kHz, fails 400kHz | Pull-up resistors too large, reduce to 2.2kΩ |
| Intermittent failures | Clock stretching unsupported by master |
| Data shifted by 1 bit | ACK bit counted as data — timing issue |
| Bus stuck LOW | Slave mid-transaction when master reset — toggle SCL 9 times to flush |
| First read byte wrong | Missing Repeated START — used STOP+START instead |

Q1: Why does I2C only need 2 wires vs SPI's 4+?

SPI dedicates separate lines per direction and uses CS pin per slave. I2C uses one bidirectional SDA for both directions and embeds slave address in protocol — all slaves share same 2 wires, no extra pins ever.

Q2: Why NACK on the last read byte?

ACK means "got it, send more." NACK on last byte tells slave "I'm done, stop sending." Without NACK, slave keeps sending indefinitely. It's the only way master signals end of read.

Q3: What happens if two masters transmit simultaneously?

Arbitration. Both monitor SDA while driving it. First one to drive HIGH while bus is LOW loses — backs off immediately. Winner completes transaction untouched. Open-drain wired-AND makes this work — LOW always dominates.

Q4: SDA changes while SCL HIGH — what does it mean?

Always START or STOP — never data. During data transmission SDA only changes when SCL is LOW. This rule makes START and STOP unambiguous and unconfutable with data bits.

Q5: Why use Repeated START instead of STOP then START for register reads?

Repeated START keeps bus ownership — no other master can grab bus between write and read phases. Also prevents slave's internal register pointer from resetting on STOP. The write-then-read is atomic with Sr.

UART Protocol CHEAT SHEET / SUMMARY

What is UART?

Asynchronous, full-duplex, point-to-point serial protocol. No shared clock, no master, no slave, no addressing — both sides are equal. Both agree on a speed (baud rate) beforehand, and each runs its own internal clock at that speed. Every byte is wrapped in a frame with START and STOP bits — that framing is what makes asynchronous communication possible.

The Lines

| Signal | Direction | Purpose |
| --- | --- | --- |
| TX | Transmitter → Receiver | Transmit data |
| RX | Receiver → Transmitter | Receive data |
| GND | Common | Voltage reference — always required |

Wiring rule — always cross-connect:

Device A TX ──────────→ Device B RX Device A RX ←────────── Device B TX Device A GND ─────────── Device B GND

TX of one side → RX of the other. TX→TX or RX→RX = nothing works. Missing GND = nothing works.

No pull-up resistors needed — push-pull drivers, unlike I2C.

The UART Frame — Core of Everything

IDLE START D0 D1 D2 D3 D4 D5 D6 D7 [PAR] STOP IDLE ‾‾‾‾‾‾¦_____¦____¦____¦____¦____¦____¦____¦____¦____¦____¦_____¦‾‾‾‾‾¦‾‾‾‾‾ ↑ ↑ Line falls Line rises (START bit) (STOP bit)

| Part | Value | Purpose |
| --- | --- | --- |
| Idle | HIGH | Line rests HIGH when nothing transmits |
| START | 1 bit, LOW | Falling edge signals frame beginning, receiver re-syncs here |
| Data | 5–8 bits | Actual data, LSB first by default |
| Parity | Optional | Even/Odd/None — basic error detection |
| STOP | 1 or 2 bits, HIGH | Returns line to idle, gives receiver processing time |

Idle is HIGH. START is LOW. The falling edge from HIGH→LOW is the synchronization trigger — receiver detects this and resets its internal bit timer.

Configuration — Must Match Both Sides Exactly

Written as: [BaudRate] [DataBits][Parity][StopBits]

115200 8N1 ← most common in embedded systems today 9600 8N1 ← legacy, debug consoles 9600 7E1 ← 7 data bits, even parity, 1 stop 9600 8O2 ← 8 data bits, odd parity, 2 stops

| Parameter | Options | Most Common |
| --- | --- | --- |
| Baud rate | 9600 / 19200 / 38400 / 57600 / 115200 / 921600 | 115200 |
| Data bits | 5 / 6 / 7 / 8 | 8 |
| Parity | None (N) / Even (E) / Odd (O) | None |
| Stop bits | 1 / 1.5 / 2 | 1 |

Baud rate mismatch = garbage data. No error message. Most common UART bug.

How Receiver Stays in Sync — Center Sampling

No shared clock means receiver must figure out bit timing on its own. Here is exactly how:

Step 1 — Receiver watches RX line, idle HIGH, waiting Step 2 — Falling edge detected → START bit beginning Step 3 — Wait 0.5 × bit period → arrive at CENTER of START bit Step 4 — Sample START bit → still LOW = valid. HIGH = glitch, ignore (false start protection) Step 5 — Wait 1 full bit period → now at CENTER of D0 → sample Step 6 — Wait 1 full bit period → CENTER of D1 → sample Step 7 — Repeat for all 8 data bits, then parity, then STOP

START D0 D1 D2 __________ ‾‾‾‾‾‾‾‾ ‾‾‾‾‾‾‾‾ ‾‾‾‾‾‾‾‾ ‾‾‾‾‾‾‾‾ ↑ ↑ ↑ ↑ Edge 0.5T 1T 1T detect (START) (D0) (D1) center center center

Why center?

Sampling at center maximizes drift tolerance. Timing can drift up to 50% of one bit period in either direction before mis-sampling. Edge sampling gives zero margin.

Baud Rate Tolerance

Both sides run independent clocks — some drift is inevitable. Drift accumulates over 10 bits (START + 8 data + STOP). Total error must stay under ~3–4% or sampling point slips outside bit window.

| Clock Source | Typical Error | Safe for UART? |
| --- | --- | --- |
| Crystal oscillator | < 0.1% | Yes, always |
| Ceramic resonator | ~0.5% | Yes |
| Internal RC oscillator | 1–5% | Risky at high baud rates |

RC oscillator + high temperature + high baud rate = intermittent UART failures. Use crystal for UART-critical designs.

Full-Duplex Nature

- TX and RX are completely independent lines. Both devices can transmit simultaneously — no coordination needed. No shared bus, no arbitration, no clock to wait for.

- Compare with I2C — half-duplex, SDA shared, only one side talks at a time. UART dedicated lines each direction = true simultaneous full-duplex.

UART vs RS-232 vs RS-485 — Common Confusion

|  | UART | RS-232 | RS-485 |
| --- | --- | --- | --- |
| What it is | Protocol + framing | Electrical standard | Electrical standard |
| Voltage | 0V / 3.3V or 5V | ±3V to ±15V | Differential ±1.5V–6V |
| Logic | HIGH=VCC LOW=GND | Inverted — HIGH=negative | Differential |
| Distance | PCB only | Up to 15m | Up to 1200m |
| Multi-drop | No | No | Yes, up to 32 nodes |

UART = the protocol (framing, baud rate, start/stop bits)
RS-232 = same UART framing, different voltage levels, needs MAX232 level shifter
RS-485 = same UART framing, differential signaling, long distance, multi-drop

Flow Control

UART has no natural way for receiver to tell transmitter to slow down — these mechanisms add that:

Hardware Flow Control — RTS/CTS (2 extra wires)

| Signal | Full Name | Meaning |
| --- | --- | --- |
| RTS | Request To Send | "I want to send, are you ready?" |
| CTS | Clear To Send | "Yes, I'm ready, go ahead" |

Transmitter only sends when CTS asserted. Receiver de-asserts CTS when buffer full → transmitter pauses. Most reliable method.

Software Flow Control — XON/XOFF (no extra wires)

- XOFF (0x13) in data stream → transmitter pauses

- XON (0x11) in data stream → transmitter resumes

Pitfall: XON/XOFF cannot be used with binary data — 0x11 or 0x13 in your data will be misread as flow control. Use hardware flow control for binary protocols.

Most embedded UARTs run with no flow control at all — rely on fast interrupt/DMA handling to drain buffer before overflow.

UART Errors

| Error | Cause | Fix |
| --- | --- | --- |
| Framing error | STOP bit is LOW — baud rate mismatch or noise | Verify baud rate both sides |
| Parity error | Parity bit mismatch — single bit flip detected | Check noise, cable, verify parity config |
| Overrun error | New byte arrived before previous byte was read | Use interrupt/DMA, add ring buffer |
| Break condition | RX held LOW longer than one full frame | Line fault or intentional (LIN bus) |

Overrun is most common in embedded — software not draining buffer fast enough. Use ring buffer in ISR.

Debug Symptom → Root Cause

| Symptom | Most Likely Cause |
| --- | --- |
| Garbage data | Baud rate mismatch — check both sides |
| Nothing received | TX→TX or RX→RX wired wrong, or missing GND |
| Works at 9600, fails at 115200 | RC oscillator drift — use crystal |
| First bytes fine, then corruption | Overrun — add ring buffer, use DMA |
| Occasional single bit errors | Line noise — add 100Ω series resistor, check GND |
| Last byte missing | TC flag not waited — UART disabled before last bit shifted out |
| Parity errors only at high temp | RC oscillator drifting with temperature |

Q: UART is async — how does receiver stay in sync with transmitter?

Re-syncs on every START bit's falling edge. From that edge receiver waits 0.5 bit periods to reach center of START, then samples every full bit period. Drift only accumulates within one 10-bit frame — fresh sync on every new frame.

Q: Why sample at center of bit, not at edge?

Center gives maximum drift tolerance — timing can slip up to 50% of one bit period either way before mis-sampling. Edge sampling gives zero margin — any drift at all causes errors.

Q: Why does UART idle HIGH and START bit is LOW?

Ensures START is always a clean falling edge from a known state. Also a disconnected/broken line floats or pulls LOW — immediately distinguishable from valid idle HIGH. It's a passive line integrity check.

Q: What is the difference between framing error and overrun error?

Framing error — STOP bit is LOW when it should be HIGH. Indicates wrong baud rate or line corruption within the frame itself.

Overrun — a new complete byte arrived before software read the previous one. Buffer overflow — frame was fine, software was too slow.

Q: Can UART support multiple slaves?

Standard UART cannot — strictly point-to-point. RS-485 uses the same UART framing on a differential bus supporting up to 32 nodes. Some MCUs also have LIN mode which adds multi-drop on top of UART framing.

SPI vs I2C vs UART — Master Comparison Table

| Feature | SPI | I2C | UART |
| --- | --- | --- | --- |
| Full Form | Serial Peripheral Interface | Inter-Integrated Circuit | Universal Asynchronous Receiver Transmitter |
| Developed By | Motorola | Philips (NXP) | — (industry standard) |
| Synchronous / Async | Synchronous | Synchronous | Asynchronous |
| Duplex | Full-duplex | Half-duplex | Full-duplex |
| Topology | One master, multi-slave | Multi-master, multi-slave | Point-to-point only |
| Number of Wires | 4 + 1 CS per extra slave | 2 (always, regardless of slaves) | 2 + GND |
| Signal Lines | SCLK, MOSI, MISO, CS | SCL, SDA | TX, RX |
| Clock | Shared — master generates | Shared — master generates | None — each side independent |
| Slave Selection | Dedicated CS pin per slave | 7-bit address in protocol | No addressing — only 2 devices |
| Addressing | None — CS does it in hardware | 7-bit (112 usable) or 10-bit | None |
| Max Speed | 50–100+ MHz | 5 MHz (FM+), 3.4 MHz (HS) | ~5 Mbps (standard UART) |
| Typical Speed | 1–50 MHz | 100 kHz / 400 kHz | 9600 / 115200 baud |
| Data Transfer | MSB first (default) | MSB first | LSB first (default) |
| Driver Type | Push-pull | Open drain | Push-pull |
| Pull-up Resistors | Not needed | Required on both lines | Not needed |
| Multi-master | Not natively supported | Yes — with arbitration | No |
| Multi-slave | Yes — one CS pin per slave | Yes — unique address per slave | No |
| Error Detection | Nonbuilt-in | ACK/NACK every byte | Parity, framing, overrun |
| Flow Control | None | Clock stretching by slave | RTS/CTS (HW) or XON/XOFF (SW) |
| Arbitration | None — bus conflict if 2 masters | Wired-AND open-drain arbitration | Applicable |
| Protocol Overhead | Minimal — pure data after CS | Address byte + ACK every byte | START + STOP bits every frame |
| Frame Structure | No framing — raw bits | START, ADDR, ACK, DATA, STOP | START, DATA, PARITY, STOP |
| Synchronization | CS falling edge + clock | START condition + clock | START bit falling edge |
| Data Direction Control | Full — MOSI and MISO simultaneously | Half — SDA one direction at a time | Full — TX and RX independent |
| Bus Capacitance Limit | No spec — PCB dependent | 400 pF max | No spec |
| Cable Distance | No spec — PCB dependent | Short — PCB traces only | Medium — RS-232 (15m) RS-485 (1200m) |
| Number of Masters | 1 (typically) | Multiple (with arbitration) | 1 per link |
| Slave Count Limit | Limited by CS pins available | 112 (7-bit) or 1024 (10-bit) | 1 only |
| Clock Stretching | Not possible — slave has no clock control | Yes — slave holds SCL LOW | Not applicable |
| Idle Line State | CS HIGH (deselected) | Both SCL and SDA HIGH | TX line HIGH |
| Dominant Bus State | Not applicable | LOW (wired-AND) | Not applicable |
| Hardware Complexity | Simple | Moderate | Simplest |
| Software Complexity | Simple | Moderate | Simple |
| Common Use Cases | Flash, SD cards, ADC, DAC, displays, sensors | Sensors, EEPROM, RTC, GPIO expanders | Debug console, GPS, Bluetooth, PC communication |
| Real Device Examples | W25Q flash, MCP3204 ADC, ILI9341 display | MPU6050, BMP280, DS3231, AT24C EEPROM | GPS modules, ESP8266, debug terminal |
| Choose When | Need speed, large data, full-duplex | Many peripherals, pin-constrained, need ACK | Simple point-to-point, debug, PC link |
| Avoid When | Pin count is tight, many slaves | Need high speed or full-duplex | Need multi-slave or long distance |