Embedded Linux for BBB

Complete Study Notes

Beginner-Friendly Reference for BeagleBone Black (BBB)

Based on Udemy Course — Enhanced & Annotated

| Topics Covered 12 Major Sections | Target Board BeagleBone Black | Level Absolute Beginner |
| --- | --- | --- |

Topics: Boot Process • U-Boot • eMMC/SD Update • Device Tree • uEnv.txt • initramfs • BusyBox • Buildroot • TFTP/NFS • Linux Source Tree

Contents

1. Linux Boot Process Overview	5

1.1 The 4-Stage Boot Chain	5

1.2 Boot Flow Diagram	6

1.3 Stage 1 — RBL (ROM Bootloader)	6

What RBL Does (in order):	6

RBL Summary Table:	8

1.4 Stage 2 — MLO / SPL (Secondary Program Loader)	8

Why Can't RBL Load U-Boot Directly?	9

What MLO Does (in order):	9

1.5 Stage 3 — U-Boot	9

U-Boot's Jobs:	9

Boot Sources Supported by U-Boot:	10

1.6 Stage 4 — Linux Kernel	10

Linux Kernel Boot Sequence (Detailed):	10

2. U-Boot — Complete Guide	11

2.1 U-Boot Source Code Structure	11

2.2 Cross Compilation — Concepts	12

Installing the Cross-Toolchain:	12

2.3 Compiling U-Boot & MLO	12

Method 1: Using the build script (easiest)	12

Method 2: Manual compilation (step by step)	13

2.4 U-Boot Environment Variables	13

Key U-Boot Commands for Environment Variables:	13

Example: Creating and Running a Custom Variable	14

2.5 Manually Booting Linux from U-Boot	14

Step-by-Step Manual Boot from MMC:	14

Decoding the UImage Header:	15

3. uEnv.txt — Automating U-Boot	18

3.1 How uEnv.txt Works	18

3.2 Complete uEnv.txt File Example and Explanation	18

Line-by-Line Explanation:	18

3.3 Creating uEnv.txt via U-Boot Shell (Serial Transfer)	19

4. SD Card Setup for Booting BBB	20

4.1 Required SD Card Partition Layout	20

4.2 Preparing the SD Card Using GParted (Ubuntu)	21

4.3 Copying Files to SD Card	22

BOOT Partition Files (copy to /dev/sdb1 mount point):	22

ROOTFS Partition Files (copy to /dev/sdb2 mount point):	22

4.4 Booting BBB from SD Card	22

5. Building the Linux Kernel from Source	23

5.1 Download the Kernel Source	23

5.2 Kernel Build Steps	23

6. Linux Device Tree	25

6.1 What Problem Does the Device Tree Solve?	25

6.2  Device Tree Files and Formats	25

6.3 Device Tree File for BBB	25

6.4 Basic Device Tree Syntax	26

6.5 How U-Boot Passes the DTB to the Kernel	26

7. initramfs — Initial RAM Filesystem	27

7.1 Is initramfs Required?	27

7.2 What Does initramfs Contain?	27

7.3 Building initramfs — Step by Step	27

Booting with initramfs from U-Boot:	28

8. Booting BeagleBone Black over Serial Port	30

8.1 How the RBL Selects UART Boot	30

How to Force UART Boot:	30

8.2 Transferring Files via xmodem	30

Complete Serial Boot Sequence:	30

9. TFTP Network Boot (Faster Development Workflow)	32

9.1 Architecture Overview	32

9.2 Setting Up TFTP Server on Ubuntu	32

9.3 TFTP Boot Commands on BBB (at U-Boot prompt)	33

9.4 Setting Up NFS Root Filesystem	34

NFS Boot Arguments (kernel bootargs):	35

10. BusyBox — Minimal Root Filesystem	36

10.1 Why BusyBox?	36

10.2 Compiling BusyBox	37

10.3 BusyBox Init System	38

Required Directories in rootfs:	39

Loading Kernel Modules at Runtime:	39

11. Buildroot — Automated Embedded Linux Build System	40

11.1 What Buildroot Can Build	40

11.2 Getting Started with Buildroot	40

11.3 Configuring and Building for BBB	41

Key menuconfig Settings for BBB:	41

11.4 Deploying Buildroot Output	42

11.5 Enabling SSH in Buildroot	43

12. Linux Source Tree & Detailed Boot Flow	44

12.1 Linux Kernel Source Tree — Key Directories	44

12.2 Detailed Boot Flow: U-Boot to Linux User Space	45

12.3 kernel_init — Finding PID 1	46

SECTION 1 — Linux Boot Process Overview

# 1. Linux Boot Process Overview

When you power on a BeagleBone Black (or any ARM-based embedded Linux board), the processor does NOT immediately run Linux. Instead, it goes through a chain of small programs called bootloaders, each one handing off to the next, until finally the Linux kernel takes over. This is called the boot chain.

📝 Note

Think of it like starting a car: the ignition key triggers the starter motor, which turns the engine, which then runs on its own. Each step is necessary and happens in a fixed sequence.

📷 Fig 1.1 — The 4-stage Linux boot chain on BBB. RBL runs from ROM (stage 1), SPL/MLO from internal SRAM (stage 2), U-Boot from DDR RAM (stage 3), then the Linux Kernel loads and mounts the Root File System (RFS) from SD/eMMC/Network (stage 4 & 5).

## 1.1 The 4-Stage Boot Chain

On the BeagleBone Black (AM335x SoC), booting happens in four distinct stages:

| Stage | Description |
| --- | --- |
| Stage 1: RBL | ROM Bootloader — hardcoded in the SoC's ROM by Texas Instruments. Runs first, always. Cannot be modified. |
| Stage 2: MLO / SPL | Memory Loader / Secondary Program Loader — a tiny program that initializes RAM and loads U-Boot. |
| Stage 3: U-Boot | Universal Bootloader — a full-featured bootloader that initializes peripherals and loads the Linux kernel. |
| Stage 4: Linux Kernel | The actual Linux operating system starts, mounts the root filesystem, and runs user applications. |

## 1.2 Boot Flow Diagram

Each box below represents a stage that runs in sequence on every power-on:

POWER ON Processor resets, start at fixed ROM address 0x20000

▼

RBL — ROM Bootloader Initializes stack, watchdog, clock → finds & copies MLO to internal SRAM

▼

MLO / SPL — Secondary Program Loader Initializes DDR RAM, UART, PLL → copies U-Boot to DDR RAM

▼

U-Boot — Universal Bootloader Initializes peripherals (USB, Ethernet, MMC) → loads kernel + DTB to DDR RAM

▼

Linux Kernel Decompresses, initializes CPU/MMU → mounts rootfs → runs /sbin/init

▼

User Space /sbin/init starts daemons, login shell appears

## 1.3 Stage 1 — RBL (ROM Bootloader)

The RBL (ROM Bootloader) is the very first piece of code that runs when the SoC gets power. It is burned permanently into the chip's internal ROM by Texas Instruments and cannot be changed.

### What RBL Does (in order):

- Sets up the CPU stack in the small internal SRAM.

- Calls the bootloader main() function.

- It initializes and starts Watchdog Timer 1 with a 3-minute timeout. If MLO is not loaded within 3 minutes, the SoC resets automatically.

- Configure the system clock using the PLL (Phase Locked Loop) engine.

- Reads boot configuration pins SYSBOOT[14:15] to determine clock source frequency.

- Scans boot devices in order (MMC, USB, UART, SPI…) to find MLO.

- Copies MLO from the boot device into the internal SRAM of the SoC.

- Jumps to the start of MLO in SRAM.

📷 Fig 1.2 — Crystal Frequencies Supported (from TI AM335x TRM, Table 26-5). This table shows how SYSBOOT[15:14] pin configuration determines which crystal frequency the RBL expects. BBB uses SYSBOOT = 01b (pin14=VCC, pin15=GND), which selects 24 MHz. The PLL then uses this 24 MHz reference to generate the 500 MHz CPU clock.

| SYSBOOT[15:14] | Crystal Frequency |
| --- | --- |
| 00b | 19.2 MHz |
| 01b (BBB default) | 24 MHz |
| 10b | 25 MHz |
| 11b | 26 MHz |

📝 Note

PLL (Phase Locked Loop) is a hardware circuit that takes a low-frequency input clock

(from a crystal oscillator — typically 24 MHz on BBB) and multiplies it up to a much

higher frequency for the CPU and peripherals. The RBL configures the PLL to run at

500 MHz for the BBB's default 24 MHz crystal.

📝 Note

SYSBOOT[14:15] are physical pins on the SoC. On BBB: pin 14 → VCC (high), pin 15 → GND (low).

This combination signals the PLL to use a 24 MHz reference and output 500 MHz.

This frequency cannot be changed during RBL — only MLO can later reconfigure it.

### RBL Summary Table:

| Task | Details |
| --- | --- |
| Stack Setup | Sets up initial CPU stack in internal SRAM for C code execution |
| Watchdog Timer | Started with 3-minute timeout as safety reset mechanism |
| Clock Init | Reads SYSBOOT pins → programs PLL → 500 MHz output |
| Boot Device Search | Scans MMC0 → MMC1 → USB → UART → SPI for MLO/SPL file |
| Copy MLO to SRAM | Copies MLO binary (max ~256 KB) into internal SRAM |
| Jump to MLO | Transfers CPU execution to MLO start address in SRAM |

⚠️ Important

The RBL always looks for a file named exactly 'MLO' on the boot device.

On a FAT-formatted SD card, this file MUST be in the root directory and named 'MLO'.

Any other name will cause the RBL to fail and the watchdog will reset the board.

## 1.4 Stage 2 — MLO / SPL (Secondary Program Loader)

MLO stands for Memory Loader (or SPL — Secondary Program Loader). It is a small program (~100–250 KB) that bridges the gap between the RBL and U-Boot. The key problem it solves: U-Boot is too large (500 KB to 1.5 MB) to fit in with the SoC's internal SRAM (~128 KB available). MLO initializes the external DDR RAM, so U-Boot has somewhere to live.

📷 Fig 1.3 — MLO vs SPL structure. Both MLO and SPL contain the same SPL executable code (shown in magenta/purple). The difference is that MLO has an Image Header prepended (shown in yellow), which contains load_addr=0x40000000 (where to load the image in SRAM) and the Size of the image. RBL reads this header first to know where to place the image in memory, then copies the SPL code there.

### Why Can't RBL Load U-Boot Directly?

| Resource | Details |
| --- | --- |
| Internal SRAM size | ~128 KB total (some reserved for stack and secure regions — effectively < 100 KB usable) |
| U-Boot typical size | 500 KB to 1.5 MB — doesn't fit in SRAM |
| DDR RAM at RBL time | DDR RAM controller not yet initialized — DDR cannot be used yet |
| MLO solution | MLO fits in SRAM, initializes DDR, then copies U-Boot to DDR |

### What MLO Does (in order):

- Initializes UART console — so you can see debug messages from this point onwards.

- Reconfigures PLL if a different clock speed is required (optional, board-specific).

- Initializes the DDR RAM controller so DDR memory becomes usable.

- Configures pin multiplexing (MUX) many SoC pins serve multiple purposes; MLO sets each pin to the function needed for booting (e.g., MMC pins).

- Reads U-Boot image from the boot device (SD card, eMMC, etc.).

- Copies U-Boot to DDR RAM at the address specified in U-Boot's header.

- Jumps to U-Boot's entry point in DDR RAM.

💡 Tip

Pin Multiplexing (MUX): A single physical pin on the SoC can be connected to different

internal hardware signals (SPI, CAN, I2C, MMC etc.) but only one at a time. MLO

configures the MUX registers to connect each pin to the right peripheral before

trying to communicate over that peripheral.

## 1.5 Stage 3 — U-Boot

U-Boot (Universal Bootloader) is an open-source, feature-rich bootloader used on millions of embedded devices worldwide. It is the third stage and the most configurable one.

### U-Boot's Jobs:

- Initializes hardware peripherals: I2C, NAND flash, Ethernet, USB, UART, MMC, etc.

- Provide an interactive command-line interface for debugging and manual booting.

- Load the Linux kernel image (uImage or zImage) from a boot source into DDR RAM.

- Load the Device Tree Blob (DTB file) into DDR RAM.

- Set up kernel boot arguments (bootargs environment variable).

- Jump to the kernel entry point.

### Boot Sources Supported by U-Boot:

U-Boot can load the kernel from: SD card (MMC0), eMMC (MMC1), USB, Ethernet (TFTP), Serial Port (xmodem), NAND flash.

## 1.6 Stage 4 — Linux Kernel

After U-Boot loads the kernel into DDR RAM and sets the boot arguments, it calls the kernel's entry point. The kernel takes over from here.

### Linux Kernel Boot Sequence (Detailed):

- U-Boot calls kernel_entry(0, machid, r2) — r2 holds the address of the DTB in DDR RAM.

- arch/arm/boot/compressed/head.S runs — sets up basic environment in Thumb/ARM mode.

- misc.c decompresses the kernel image (zImage is a compressed kernel).

- arch/arm/kernel/head.S runs — performs CPU-specific initialization.

- head.S validates the processor architecture, sets up page tables, initializes and enables the MMU (virtual memory).

- head.S calls start_kernel() in init/main.c.

- start_kernel() initialises all kernel subsystems: memory, scheduler, IRQ, filesystems…

- start_kernel() calls rest_init() which creates two kernel threads: kernel_init and kthreadd.

- The scheduler starts running. The idle task (while(1)) runs when nothing else is scheduled.

- kernel_init frees the memory used for boot-time init functions (marked __init).

- kernel_init tries to execute the first userspace program in this order: /sbin/init → /etc/init → /bin/init → /bin/sh. The first one that exists and runs successfully becomes PID 1.

📝 Note

MMU = Memory Management Unit. It translates 'virtual addresses' that programs use

into 'physical addresses' that actually exist in RAM. Before MMU is enabled, every

address is a direct physical address. After MMU is on, processes get isolated address

spaces and can't accidentally overwrite each other's memory.

SECTION 2 — U-Boot Deep Dive

# 2. U-Boot — Complete Guide

U-Boot is the most important bootloader in the embedded Linux world. This section covers its source structure, environment variables, commands, and how to compile it from scratch.

## 2.1 U-Boot Source Code Structure

Clone URL: https://openbeagle.org/beagleboard/u-boot/-/tree/v2022.04-bbb.io-am335x-am57xx

| Directory/File | Purpose |
| --- | --- |
| arch/ | Architecture-specific code (ARM, MIPS, x86, RISC-V, etc.) |
| arch/arm/cpu/ | ARM CPU-specific code |
| arch/arm/cpu/arm7/start.S | Entry point: the assembly file that jumps from ROM to SPL |
| arch/arm/lib/bootm.c | Kernel boot code — picks kernel from RAM and hands over control |
| board/ | Board-specific files for hundreds of hardware platforms |
| board/ti/am335x/ | Files specific to the BeagleBone Black (AM335x SoC) |
| configs/ | Default configuration files for each supported board |
| configs/am335x_boneblack_defconfig | Default config file for BBB — used during build |
| include/ | Header files and board configuration headers |
| common/ | Common code: command implementation, environment handling |
| drivers/ | Device drivers: MMC, USB, Ethernet, serial, etc. |
| Makefile | Top-level build system entry point |

## 2.2 Cross Compilation — Concepts

Since your development PC runs on x86/x64 but BBB uses ARM, you cannot compile U-Boot with your PC's native GCC. You need a cross-compiler — a compiler that runs on x86 but produces ARM binary code.

| Term | Meaning |
| --- | --- |
| Host machine | Your PC (x86_64 Linux) |
| Target machine | BeagleBone Black (ARM Cortex-A8) |
| Cross compiler | GCC that runs on host but generates code for target |
| CROSS_COMPILE prefix | arm-linux-gnueabihf- (hard-float ABI) or arm-linux-gnueabi- |
| Toolchain | Set of tools: compiler (gcc), linker (ld), assembler (as), debugger (gdb) |

### Installing the Cross-Toolchain:

Download from: https://releases.linaro.org/components/toolchain/binaries/latest-7/arm-linux-gnueabi/

# Extract the toolchain

tar -xf gcc-linaro-7.5.0-2019.12-x86_64_arm-linux-gnueabi.tar.xz

# Add to PATH permanently — edit your shell profile:

vi ~/.bashrc

# Add this line at the end:

export PATH=$PATH:/home/user/gcc-linaro-7.5.0-2019.12-x86_64_arm-linux-gnueabi/bin

# Apply changes:

source ~/.bashrc

# Verify installation:

arm-linux-gnueabihf-gcc --version

## 2.3 Compiling U-Boot & MLO

### Method 1: Using the build script (easiest)

# Give execution permission

sudo chmod +x build-am335x.sh

# Run the script

sudo ./build-am335x.sh

### Method 2: Manual compilation (step by step)

Step 1: Clean previous build artifacts

make CROSS_COMPILE=arm-linux-gnueabihf- distclean

This deletes all previously compiled object files so you start fresh.

Step 2: Apply board default configuration

make CROSS_COMPILE=arm-linux-gnueabihf- am335x_boneblack_defconfig

This reads the defconfig file from configs/ and writes a .config file.

Output: 'configuration written to .config'

Step 3: Optional: Open menuconfig for customisation

make CROSS_COMPILE=arm-linux-gnueabihf- menuconfig

A text-based GUI opens. You can enable/disable features. Press Escape when done.

Step 4: Compile with parallel jobs

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- -j4 # for 4-core CPU

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- -j8 # for 8-core CPU

The -j flag tells 'make' to run N compilation jobs in parallel (speeds up build).

After successful compilation, these files are generated:

• u-boot.img — the U-Boot bootloader binary

• MLO — the SPL/Secondary Program Loader

• spl/u-boot-spl.bin — SPL binary (same as MLO)

## 2.4 U-Boot Environment Variables

U-Boot maintains a set of environment variables that control its behaviour. These can be viewed, created, and modified at the U-Boot prompt.

### Key U-Boot Commands for Environment Variables:

| Command | What it Does |
| --- | --- |
| printenv | Lists ALL environment variables and their values |
| printenv bootargs | Shows the value of a specific variable (bootargs in this example) |
| setenv serverip 192.168.7.1 | Creates or updates an environment variable |
| setenv bootargs console=ttyO0,115200 | Sets the kernel boot arguments |
| run bootcmd | Executes the commands stored inside the bootcmd variable |
| saveenv | Saves all current environment variables to persistent storage |
| env import -t 0x82000000 36 | Imports environment variables from memory address 0x82000000 (36 bytes) |

### Example: Creating and Running a Custom Variable

# Create a variable that holds a command:

setenv my_mmc_list 'mmc list'

# Check it was saved:

printenv my_mmc_list

# Output: my_mmc_list=mmc list

# Run the command stored in the variable:

run my_mmc_list

# Output:

# OMAP SD/MMC: 0 (SD)     <-- SD card slot

# OMAP SD/MMC: 1          <-- eMMC storage

## 2.5 Manually Booting Linux from U-Boot

You can boot Linux manually from the U-Boot prompt without any uEnv.txt file. This is useful for debugging. Press any key when you see 'Hit any key to stop autoboot 3' to enter the U-Boot shell.

### Step-by-Step Manual Boot from MMC:

# Step 1: List of available MMC devices

mmc list

# OMAP SD/MMC: 0 (SD)  <-- mmc 0 = SD card

# OMAP SD/MMC: 1         <-- mmc 1 = eMMC

# Step 2: Load kernel image into DDR RAM

# Syntax: load mmc <device>:<partition> <address> <filename>

load mmc 0:1 0x82000000 uImage

# '0:1' means device 0 (SD card), partition 1 (BOOT partition, FAT)

# '0x82000000' is the DDR RAM target address

# Step 3: Load Device Tree Blob (DTB) into DDR RAM

load mmc 0:1 0x88000000 am335x-boneblack.dtb

# Step 4: Set kernel boot arguments

setenv bootargs console=ttyO0,115200 root=/dev/mmcblk0p2 rw

# ttyO0 = UART0 serial console at 115200 baud

# root=/dev/mmcblk0p2 = mount partition 2 of SD card as root filesystem

# Step 5: Boot the kernel

# Syntax: bootm <kernel_addr> - <dtb_addr>

bootm 0x82000000 - 0x88000000

# The '-' in the middle is a placeholder for initramfs (not used here)

📝 Note

Address Map for BBB DDR RAM:

0x82000000 → Load kernel image (uImage) here

0x88000000 → Load Device Tree Blob (.dtb) here

0x88080000 → Load initramfs here (if used)

These are conventions — you can use other addresses if they don't overlap.

### Decoding the UImage Header:

A uImage is a zImage (compressed kernel) with a 64-byte U-Boot header prepended. You can inspect this header:

uImage Structure

s

📷 Fig 2.1 — uImage structure. A uImage (ELF binary format) is created by taking the standard zImage (the compressed Linux kernel ELF binary) and prepending a 64-byte U-Boot image header using the 'mkImage' command. The header tells U-Boot: where to load the image in DDR RAM, the image size, CRC checksum, architecture type, and entry point address.

📷 Fig 2.2 — U-Boot Linux Image Header C structure (image_header_t). This is the actual C struct definition from U-Boot source code (include/image.h). Each field is a 32-bit or 8-bit integer stored in big-endian (network byte order). ih_magic identifies the file as a U-Boot image, ih_hcrc verifies the header itself, ih_dcrc verifies the data payload, ih_load is where to put the image in RAM, ih_ep is the first instruction to run (entry point), ih_name is a 32-character human-readable name.

Manually Reading the uImage Header at U-Boot Prompt

After loading uImage to DDR RAM at 0x82000000, you can inspect its header two ways:

📷 Fig 2.3 — Using 'md' (memory display) command at U-Boot prompt to read raw header bytes. Command: 'md 0x82000000 4' displays 4 words (16 bytes) starting at 0x82000000. The four highlighted values are (in order): Magic Number (0x56190527 — this is the U-Boot magic 'IH_MAGIC'), CRC Checksum of the header, Image Creation Timestamp (Unix timestamp), and Size of Image in bytes. These are raw hex values in big-endian format.

📷 Fig 2.4 — Extended 'md 0x82000000 10' output showing 16 words (64 bytes = full header). The highlighted text on the right side shows 'Angstrom/3.8.13/beaglebone’ this is the ih_name field decoded as ASCII. Reading the rows: Row 1 (magic, CRC, timestamp, size), Row 2 (load address 0x80008000, entry point 0x80008000, data CRC, OS/arch/type/comp bytes), Rows 3-4 (the 32-byte image name string).

📷 Fig 2.5 — Using 'imi 0x82000000' command — the Image Info command. This is the proper way to decode the U-Boot header. It prints all fields in human-readable format: Image Name, Image Type (ARM Linux Kernel), Data Size (4.1 MiB), Load Address (0x80008000), Entry Point (0x80008000), and Checksum verification result (OK). This is much easier to read than raw hex from 'md'.

# The two commands to read uImage header:

md 0x82000000 4       # Raw hex dump of first 16 bytes (manual decoding)

imi 0x82000000        # Formatted image info (recommended — much clearer)

U-Boot Boot Log — What Success Looks Like

# After loading uImage to DDR RAM:

imi 0x82000000

# Example output:

# Image Name:   Linux-4.4.62-ti-r99

# Created:      2017-xx-xx

# Image Type:   ARM Linux Kernel Image (uncompressed)

# Data Size:    11334208 Bytes = 10.8 MiB

# Load Address: 80008000

# Entry Point: 80008000

# Verifying Checksum ... OK

📷 Fig 2.6 — Successful U-Boot kernel boot log in minicom. Reading line by line: '## Booting kernel from Legacy Image at 82000000' — U-Boot found the kernel at the DDR address; Image Name, Type, Size, Load Address, Entry Point are printed from the uImage header; 'Verifying Checksum ... OK' — CRC check passed; '## Flattened Device Tree blob at 88000000' — DTB found at its DDR address; 'Loading Kernel Image ... OK'; 'Using Device Tree in place at 88000000'; 'Starting kernel ...' — this is U-Boot's

⚠️ Important

'Starting kernel ...' is the last message printed by U-Boot.

Everything after that comes from the Linux kernel itself.

If the screen goes blank after 'Starting kernel ...' it means the kernel

crashed or the console (ttyO0) is not configured correctly in bootargs.

SECTION 3 — uEnv.txt File

# 3. uEnv.txt — Automating U-Boot

Instead of typing boot commands manually at the U-Boot prompt every time, you can create a file called uEnv.txt and place it on the SD card's BOOT partition. U-Boot reads this file automatically and executes the commands inside it. This is how BBB automates the Linux boot process.

📝 Note

uEnv.txt must be placed in the BOOT partition (FAT formatted, partition 1) of the SD card,

NOT in the ROOTFS partition. U-Boot can only read FAT partitions; it cannot read ext3/ext4.

## 3.1 How uEnv.txt Works

When U-Boot starts its boot process (the 'boot' command), it automatically looks for a file named uEnv.txt on the boot device. If found, it reads the variables defined in the file and adds them to U-Boot's environment, then executes the 'uenvcmd' variable if it exists.

## 3.2 Complete uEnv.txt File Example and Explanation

console=ttyO0,115200n8

ipaddr=192.168.7.2

serverip=192.168.7.1

loadaddr=0x82000000

fdtaddr=0x88000000

loadfromsd=load mmc 0:2 ${loadaddr} /boot/uImage;load mmc 0:2 ${fdtaddr} /boot/am335x-boneblack.dtb

linuxbootargs=setenv bootargs console=${console} root=/dev/mmcblk0p2 rw

uenvcmd=setenv autoload no; run loadfromsd; run linuxbootargs; bootm ${loadaddr} - ${fdtaddr}

### Line-by-Line Explanation:

| Variable | Explanation |
| --- | --- |
| console=ttyO0,115200n8 | Sets the Linux kernel console to UART0 at 115200 baud, 8N1 (no parity, 1 stop bit). 'n8' means 'no parity, 8 data bits'. |
| ipaddr=192.168.7.2 | Sets the IP address of the BBB board (used for TFTP/NFS networking). |
| serverip=192.168.7.1 | Sets the IP address of the host PC (TFTP/NFS server). |
| loadaddr=0x82000000 | A variable storing the DDR address where the kernel image will be loaded. |
| fdtaddr=0x88000000 | A variable storing the DDR address where the Device Tree Blob (DTB) will be loaded. 'fdt' = Flattened Device Tree. |
| loadfromsd=load mmc 0:2... | Loads uImage from SD card (mmc 0, partition 2) to loadaddr, then loads DTB to fdtaddr. ';' chains two commands. |
| linuxbootargs=setenv bootargs... | Builds the kernel command line tells the kernel to use ttyO0 for console, mount mmcblk0p2 (SD partition 2) as root, read-write. |
| uenvcmd=... | The main auto-execute variable. Disables auto DHCP, runs loadfromsd, sets bootargs, then boots the kernel. |

📝 Note

mmc 0:2 breakdown:

mmc 0 = First MMC device (SD card in the card slot)

mmc 1 = Second MMC device (eMMC soldered to board)

0:2 = Device 0, Partition 2 (the ext3 ROOTFS partition)

0:1 = Device 0, Partition 1 (the FAT BOOT partition)

mmcblk0p2 = Linux device name for SD card partition 2 (same as 0:2)

## 3.3 Creating uEnv.txt via U-Boot Shell (Serial Transfer)

If you need to test uEnv.txt over serial without an SD card, you can create the file on your PC, transfer it via xmodem/ymodem protocol, and import it into U-Boot memory:

# Create uEnv.txt on your PC with content like:

mypcip=setenv serverip 192.168.1.2

# In minicom, transfer the file using ymodem protocol:

# Press: Ctrl+A, then S → select 'ymodem' → navigate to file → Enter

# After transfer, note the address where it was loaded

# Then import its variables into U-Boot environment:

env import -t 0x82000000 36

# '36' = number of bytes in the file (check the transfer output)

# Verify the variable was imported:

printenv mypcip

# Output: mypcip=setenv serverip 192.168.1.2

# Execute the variable:

run mypcip

printenv serverip

# Output: serverip=192.168.1.2

SECTION 4 — SD Card Setup & Booting from SD

# 4. SD Card Setup for Booting BBB

To boot BBB from an SD card (instead of the default eMMC), you need to prepare the SD card with two specific partitions and copy the correct boot files to each partition.

## 4.1 Required SD Card Partition Layout

| Partition | Details |
| --- | --- |
| Partition 1: BOOT | Format: FAT16 or FAT32 ¦ Label: BOOT ¦ Size: 1 GB ¦ Flag: bootable ¦ Contents: MLO, u-boot.img, uEnv.txt |
| Partition 2: ROOTFS | Format: ext3 ¦ Label: ROOTFS ¦ Size: Remaining space ¦ Contents: Extracted root filesystem (all Linux files) |

📷 Fig 4.1 — µSD Card Partitions diagram. The SD card must be divided into exactly two partitions: Partition 1 (BOOT) — formatted as FAT filesystem, holds boot images (MLO, u-boot.img, uEnv.txt). This is the partition U-Boot reads because U-Boot only understands FAT. Partition 2 (ROOTFS) — formatted as EXT3 or EXT4, holds the complete Linux root filesystem (all system files, libraries, init scripts, user applications). The arrow at the bottom points to Partition 1 with the label 'This partition holds boot images'.

📝 Note

Why two partitions?

U-Boot can only read FAT filesystems (BOOT partition). The Linux kernel needs an

ext3/ext4 filesystem for the root filesystem (ROOTFS partition). They must be separate

because each has different filesystem format requirements.

## 4.2 Preparing the SD Card Using GParted (Ubuntu)

- Connect SD card reader to PC.

- Open a terminal and run: dmesg to identify the SD card device name (e.g., /dev/sdb).

- Install GParted if not already: sudo apt-get install gparted

- Open GParted and select the SD card from the device drop-down.

- Delete all existing partitions: right-click each → Delete.

- Create BOOT partition: right-click unallocated → New → Filesystem: fat16 → Label: BOOT → Size: 1024 MB → Add.

- Create ROOTFS partition: right-click remaining unallocated → New → Filesystem: ext3 → Label: ROOTFS → Size: all remaining → Add.

- Click Apply (green checkmark) to write changes.

- Right-click BOOT partition → Manage Flags → tick 'boot' → Close.

# Alternative: Use command line with fdisk (for advanced users)

sudo fdisk /dev/sdb

# Then follow n (new), p (primary), partition numbers, sizes

# Then: t (type), change partition 1 to FAT16 (type 6)

# Then: a (bootable flag on partition 1)

# Then: w (write and exit)

# Format the partitions:

sudo mkfs.vfat -F 16 -n BOOT /dev/sdb1

sudo mkfs.ext3 -L ROOTFS /dev/sdb2

GParted Result — Correct Partition Configuration

📷 Fig 4.2 — GParted showing the correctly partitioned SD card (/dev/sdb, 7.28 GiB total). The graphical bar at the top shows the two partitions visually. The table confirms: /dev/sdb1 = fat16 filesystem, labelled BOOT, 1013 MiB, with the 'boot' flag set (critical — RBL only boots from a partition with this flag). /dev/sdb2 = ext3 filesystem, labelled ROOTFS, 6.29 GiB remaining space. The 'boot' flag on sdb1 is what tells the RBL this is the bootable partition.

## 4.3 Copying Files to SD Card

### BOOT Partition Files (copy to /dev/sdb1 mount point):

| File | Source / Notes |
| --- | --- |
| MLO | Rename 'MLO-beaglebone-2013.04' to 'MLO'. This exact name is required. |
| u-boot.img | Rename 'u-boot-beaglebone-2013.04-r0.img' to 'u-boot.img'. Exact name required. |
| uEnv.txt | Your custom boot configuration file (optional but recommended). |

### ROOTFS Partition Files (copy to /dev/sdb2 mount point):

# Mount ROOTFS partition

sudo mount /dev/sdb2 /mnt/rootfs

# Extract root filesystem (requires root for correct permissions)

sudo tar -xf Angstrom-systemd-image-eglibc-ipk-v2012.12-beagleboard.rootfs.tar.xz -C /mnt/rootfs

# Unmount when done

sudo umount /mnt/rootfs

⚠️ Important

Always use 'sudo' when copying to SD card partitions.

Regular user permissions will cause 'permission denied' errors and the files won't copy correctly.

## 4.4 Booting BBB from SD Card

By default, BBB boots from the internal eMMC storage. To force it to boot from the SD card, use the S2 (boot) button:

- Insert the prepared SD card into the BBB SD card slot.

- Connect the UART-USB debug cable to your PC (for minicom serial output).

- Open minicom on your PC: minicom -D /dev/ttyUSB0 -b 115200

- Power off the BBB by long pressing the power button until the power LED turns off.

- Press and HOLD the S2 button (boot button, located near the SD card slot).

- While holding S2, press the power button to power on the board.

- Release S2 after a few seconds — the board will now boot from SD card instead of eMMC.

📝 Note

S2 is the small button next to the SD card slot on the BBB board.

It forces RBL to change its boot sequence to check the SD card first.

Without pressing S2, the board always boots from eMMC by default.

SECTION 5 — Linux Kernel Build

# 5. Building the Linux Kernel from Source

The Linux kernel is the core of the operating system. When building for BBB, we cross-compile it on our x86_64 PC to generate an ARM binary. The output is a 'uImage' — a compressed kernel with a U-Boot header.

## 5.1 Download the Kernel Source

# Clone the BBB-specific kernel (v5.10.168 with TI real-time patches):

git clone https://github.com/beagleboard/linux.git

cd linux

git checkout v5.10.168-ti-rt-r76

## 5.2 Kernel Build Steps

Step 1: Clean previous build artifacts

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- distclean

Step 2: Apply board default configuration

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- bb.org_defconfig

For kernel v4.11 and later, use: omap2plus_defconfig

This generates the .config file with all the driver selections for BBB.

Step 3: Customise kernel configuration (optional)

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- menuconfig

In the menuconfig UI:

[*] = built into kernel (compiled in)

[M] = loadable module (compiled as .ko file, loaded at runtime)

[ ] = disabled (not compiled at all)

Removing unnecessary drivers reduces the final kernel size.

Step 4: Compile the kernel and Device Tree

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- uImage dtbs LOADADDR=0x80008000 -j4

uImage = compressed kernel with U-Boot header

dtbs = Device Tree Blobs for all supported boards

LOADADDR = the physical RAM address where the kernel will run

Output: arch/arm/boot/uImage and arch/arm/boot/dts/am335x-boneblack.dtb

Step 5: Compile loadable kernel modules

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- -j4 modules

Compiles all drivers selected as [M] in menuconfig into .ko files.

These files are needed to load device drivers at runtime.

Step 6: Install modules to root filesystem

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- INSTALL_MOD_PATH=<path/to/RFS> modules_install

Copies all .ko files into <RFS>/lib/modules/<kernel-version>/

Replace <path/to/RFS> with your root filesystem path, e.g., /srv/nfs/bbb

📝 Note

LOADADDR=0x80008000 — this is where the kernel will be placed in physical RAM.

The U-Boot 'load' command copies the uImage binary to the DDR address 0x82000000,

but the kernel itself decompresses and relocates to 0x80008000 before running.

These are two different addresses and both must be correct for the kernel to boot.

SECTION 6 — Linux Device Tree

# 6. Linux Device Tree

The Device Tree is a data structure that describes the hardware of a board to the Linux kernel. Before Device Trees existed, hardware descriptions were hardcoded into the kernel source code — which meant maintaining thousands of board-specific kernel forks. Device Trees solved this problem.

## 6.1 What Problem Does the Device Tree Solve?

On x86 PCs, the BIOS/UEFI firmware discovers hardware automatically (via PCI, USB enumeration). On ARM embedded boards, there is no such automatic discovery — every peripheral (UART, SPI, I2C, GPIO, ethernet) is at a fixed memory address and the kernel must be told exactly where each one is.

The Device Tree is essentially a hardware description file. It says: 'There is a UART at address 0x44E09000, connected to interrupt 72, running at 48 MHz clock.' The kernel reads this file and configures the appropriate driver.

## 6.2  Device Tree Files and Formats

| Extension | Description |
| --- | --- |
| .dts | Device Tree Source — human-readable text file that describes the hardware. Written by developers. |
| .dtsi | Device Tree Source Include — a shared fragment included by multiple .dts files (e.g., common SoC peripherals). |
| .dtb | Device Tree Blob — binary compiled version of the .dts file. This is what U-Boot loads and passes to the kernel. |
| dtc | Device Tree Compiler — tool that compiles .dts → .dtb or decompiles .dtb → .dts |

## 6.3 Device Tree File for BBB

The main Device Tree file for BeagleBone Black is:

arch/arm/boot/dts/am335x-boneblack.dts

It includes the SoC-level file:

arch/arm/boot/dts/am335x.dtsi  (AM335x SoC peripherals)

arch/arm/boot/dts/am33xx.dtsi  (AM33xx family peripherals)

## 6.4 Basic Device Tree Syntax

/ {

/* Root node — represents the entire system */

model = "TI AM335x BeagleBone Black";

compatible = "ti,am335x-bone-black", "ti,am335x-bone", "ti,am33xx";

cpus {

cpu@0 {

compatible = "arm,cortex-a8";

};

};

memory@80000000 {

device_type = "memory";

reg = <0x80000000 0x20000000>;  /* 512 MB DDR starting at 0x80000000 */

};

uart0: serial@44e09000 {

compatible = "ti,am3352-uart";

reg = <0x44e09000 0x2000>;   /* register base address, size */

interrupts = <72>;

status = "okay";             /* enabled */

};

};

📝 Note

The 'compatible' property is how the kernel matches a device node to a driver.

The kernel's driver database is searched for a driver that lists 'ti,am3352-uart'

in its compatible table. When found, that driver is used to control the UART.

status = 'okay' means the device is enabled. status = 'disabled' means it's off.

## 6.5 How U-Boot Passes the DTB to the Kernel

U-Boot loads the .dtb file into DDR RAM at fdtaddr (typically 0x88000000) and then passes this address to the kernel as parameter r2 when calling kernel_entry(0, machid, r2). The kernel reads the hardware description from this memory location during boot.

# U-Boot: Load DTB and pass it to kernel

load mmc 0:2 ${fdtaddr} /boot/am335x-boneblack.dtb

bootm ${loadaddr} - ${fdtaddr}

#          ^kernel   ^ DTB address

SECTION 7 — initramfs (Initial RAM Filesystem)

# 7. initramfs — Initial RAM Filesystem

initramfs (Initial RAM File System) is a temporary, compressed root filesystem that is loaded into RAM during early kernel boot. It gives the kernel a minimal working environment before the actual root filesystem (on SD card / eMMC / NFS) is mounted.

## 7.1 Is initramfs Required?

No — initramfs is optional. Most desktop Linux distributions use it (to load drivers needed for the real disk). In embedded Linux, it is optional and commonly used when:

- The real root filesystem requires a driver that is not compiled into the kernel.

- You want to boot entirely in RAM (no SD card / eMMC needed).

- You need a recovery environment.

| With initramfs | Without initramfs |
| --- | --- |
| Kernel boots → loads initramfs → runs /init from RAM → mounts real rootfs | Kernel boots → directly mounts real rootfs (SD/eMMC) → runs /sbin/init |
| Useful: loads missing drivers before mounting real rootfs | Simpler: fewer components, faster if all drivers are built-in |
| Required if root-disk driver is a loadable module | Works fine if all needed drivers are compiled into the kernel |

## 7.2 What Does initramfs Contain?

- Device drivers (as loadable modules if needed)

- Firmware files for hardware initialisation

- /init script — the first program run from initramfs

- Essential utilities (mount, pivot_root, switch_root)

- Boot scripts and configuration files

## 7.3 Building initramfs — Step by Step

Step 1: Create cpio archive from your root filesystem directory

cd <your_rootfs_directory>

find . ¦ cpio -H newc -o > ../initramfs.cpio

'cpio' = Copy In/Out. '-H newc' = new (SVR4) portable format. '-o' = output mode.

This creates a single archive file containing all files in the current directory.

Step 2: Compress the cpio archive with gzip

gzip < ../initramfs.cpio > ../initramfs.gz

Compresses the archive significantly (typically 3–5x smaller).

Step 3: Install U-Boot tools

sudo apt-get install u-boot-tools

This installs the 'mkimage' utility needed for Step 4.

Step 4: Add U-Boot header with mkimage

mkimage -A arm -O linux -T ramdisk -C none -a 0x80800000 -n "Root Filesystem" -d ../initramfs.gz ../initramfs

-A arm = target architecture

-O linux = operating system

-T ramdisk = image type is ramdisk (initramfs)

-C none = not compressed at this stage (already gzipped)

-a 0x80800000 = load address in DDR RAM

-n "Root ..." = image name/description

-d ../initramfs.gz = input file

Output: 'initramfs' binary ready to be loaded by U-Boot

### Booting with initramfs from U-Boot:

# Load kernel image

loadx 0x82000000       # or: load mmc 0:1 0x82000000 uImage

# Load DTB

loadx 0x88000000       # or: load mmc 0:1 0x88000000 am335x-boneblack.dtb

# Load initramfs

loadx 0x88080000       # or: load mmc 0:1 0x88080000 initramfs

# Set boot args: root=/dev/ram0 tells kernel to use initramfs as root

setenv bootargs console=ttyO0,115200 root=/dev/ram0 rw initrd=0x88080000

# Boot: note all three addresses are given

bootm 0x82000000 0x88080000 0x88000000

#     ^kernel     ^initramfs  ^dtb

SECTION 8 — Booting BBB via Serial Port

# 8. Booting BeagleBone Black over Serial Port

If no SD card, USB, or SPI device is available, the BBB's RBL will fall back to UART boot mode. This allows you to load MLO, U-Boot, kernel, and DTB directly from your PC to the BBB using the xmodem serial transfer protocol via minicom.

## 8.1 How the RBL Selects UART Boot

The RBL scans boot sources in this fixed order: SPI → MMC0 (SD card) → USB → UART. When all other sources fail or are absent, it starts printing 'CCC...' on the UART — this is the xmodem CRC handshake signal, indicating it is ready to receive data.

### How to Force UART Boot:

- Remove SD card from BBB.

- Do NOT connect any USB storage to the P4 mini-USB port.

- Connect only the UART debug cable (UART0 RX/TX/GND pins) to your PC.

- Open minicom on PC (baud rate 115200, 8N1).

- Hold the S2 (boot) button and press power button. You will see 'CCCC...' in minicom.

## 8.2 Transferring Files via xmodem

Once you see 'CCC...' in minicom, transfer files using the xmodem protocol:

# In minicom, to send a file using xmodem:

# Press: Ctrl+A, then release, then press S

# A menu appears. Select 'xmodem' using arrow keys, press Enter

# Navigate to your file, press Space to select it, press Enter to start transfer

### Complete Serial Boot Sequence:

- See 'CCC...' — RBL is waiting. Send SPL binary (sp.bin or MLO.byteswap) via xmodem. SPL is written to internal SRAM.

- SPL loads and prints messages. It now waits for U-Boot. Send u-boot.img via xmodem. U-Boot is written to DDR RAM.

- U-Boot starts. Press Space bar to stop autoboot and get the U-Boot prompt.

- Transfer kernel image to DDR RAM:

loadx 0x82000000

# Then send uImage via xmodem from minicom

- Transfer DTB:

loadx 0x88000000

# Then send am335x-boneblack.dtb via xmodem

- Transfer initramfs (optional):

loadx 0x88080000

# Then send initramfs via xmodem

- Set boot arguments and boot:

setenv bootargs console=ttyO0,115200 root=/dev/ram0 rw initrd=0x88080000

bootm 0x82000000 0x88080000 0x88000000

💡 Tip

Serial boot is mostly useful for recovery or first-time board bring-up.

For regular development, TFTP boot (Section 9) is much faster because

you don't have to manually transfer each file — TFTP downloads them over Ethernet.

SECTION 9 — TFTP Network Boot

# 9. TFTP Network Boot (Faster Development Workflow)

TFTP (Trivial File Transfer Protocol) allows U-Boot on BBB to download the kernel, DTB, and initramfs from a TFTP server running on your host PC over an Ethernet cable. This is the fastest way to test new kernel or rootfs changes during development — no SD card re-flashing needed.

Hardware Connection

📷 Fig 9.1 — TFTP network connection diagram. Linux Host PC (left, IP=192.168.7.1) acts as the TFTP server — it runs tftpd and the kernel files are stored in /var/lib/tftpboot on this machine. BeagleBone Black (right, IP=192.168.7.2) acts as the TFTP client — U-Boot on BBB connects to the host PC and downloads the files. Both are connected with a direct Ethernet cable (crossover or standard, depending on your NICs). The bidirectional arrow shows TFTP data flows in both directions (request from BBB, data from PC).

## 9.1 Architecture Overview

The big picture: store MLO, u-boot.img, and uEnv.txt on eMMC to boot up to U-Boot, then transfer kernel and DTB from your PC using TFTP and mount the root filesystem from your PC using NFS.

| Component | Location |
| --- | --- |
| MLO + u-boot.img | eMMC or SD card (FAT partition) |
| uEnv.txt | eMMC or SD card (FAT partition) |
| uImage (kernel) | Host PC TFTP server directory: /var/lib/tftpboot/ |
| am335x-boneblack.dtb | Host PC TFTP server directory: /var/lib/tftpboot/ |
| initramfs | Host PC TFTP server directory: /var/lib/tftpboot/ |
| Root Filesystem | Host PC NFS server directory: /srv/nfs/bbb/ |

## 9.2 Setting Up TFTP Server on Ubuntu

Step 1: Update and install TFTP daemon

sudo apt-get update

sudo apt-get install tftpd-hpa

Step 2: Configure the TFTP server

sudo vim /etc/default/tftpd-hpa

Add/edit these lines:

TFTP_USERNAME="tftp"

TFTP_DIRECTORY="/var/lib/tftpboot"

TFTP_ADDRESS=":69"

TFTP_OPTIONS="--create --secure"

Step 3: Create and configure the TFTP directory

sudo mkdir -p /var/lib/tftpboot

sudo chown tftp:tftp /var/lib/tftpboot

sudo chmod -R 777 /var/lib/tftpboot

Copy uImage, am335x-boneblack.dtb, and initramfs to this directory.

Step 4: Start the TFTP service

sudo systemctl start tftpd-hpa

sudo systemctl enable tftpd-hpa # start on boot

## 9.3 TFTP Boot Commands on BBB (at U-Boot prompt)

# Set network addresses

setenv ipaddr   192.168.27.2    # BBB's IP address

setenv serverip 192.168.27.1    # Host PC's IP address

# Test connectivity

ping 192.168.27.1

# Download files from TFTP server to DDR RAM

tftpboot 0x82000000 uImage

tftpboot 0x88000000 am335x-boneblack.dtb

tftpboot 0x88080000 initramfs

# Set boot arguments

setenv bootargs console=ttyO0,115200 root=/dev/ram0 rw initrd=0x88080000

# Boot

bootm 0x82000000 0x88080000 0x88000000

## 9.4 Setting Up NFS Root Filesystem

NFS (Network File System) allows BBB to mount its root filesystem from the host PC over the network. Combined with TFTP, this gives you a fully network-booted BBB with no local storage required — perfect for development.

Step 1: Install NFS server on host PC

sudo apt-get install nfs-kernel-server

Step 2: Configure NFS exports

sudo vim /etc/exports

Add this line:

/srv/nfs/bbb 192.168.27.2(rw,sync,no_subtree_check,no_root_squash)

This exports /srv/nfs/bbb directory to the BBB at 192.168.27.2

📷 Fig 9.2 — /etc/exports file entry for NFS server. This single line exports the /srv/nfs/bbb directory to the BBB (IP 192.168.7.2) with options: rw (read-write access), sync (write operations complete before returning), no_root_squash (allows root on BBB to have root access to NFS files — needed for device files and permissions), no_subtree_check (performance improvement). After editing, run 'sudo exportfs -rv' to apply changes.

Step 3: Configure host PC network interface

sudo vim /etc/network/interfaces

Add entry for your Ethernet interface with static IP 192.168.27.1

Step 4: Apply NFS settings

sudo exportfs -a

sudo exportfs -rv

sudo service nfs-kernel-server restart

sudo service nfs-kernel-server status

### NFS Boot Arguments (kernel bootargs):

setenv bootargs console=ttyO0,115200 root=/dev/nfs nfsroot=192.168.27.1:/srv/nfs/bbb,v3 rw ip=192.168.27.2

💡 Tip

TFTP + NFS workflow for daily development:

1. Edit source code on your PC

2. Compile new kernel or application

3. Copy binary to /var/lib/tftpboot/ or /srv/nfs/bbb/

4. Reboot BBB — it downloads fresh binaries automatically

No SD card re-flashing ever needed!

SECTION 10 — BusyBox

# 10. BusyBox — Minimal Root Filesystem

BusyBox is a single executable binary that contains minimised versions of over 300 Unix/Linux command-line utilities. It replaces the need for separate binaries like ls, cp, mv, grep, sed, awk, etc. BusyBox is the standard way to create a minimal root file system for embedded Linux products.

## 10.1 Why BusyBox?

| Standard Linux (PC) | BusyBox (Embedded) |
| --- | --- |
| /bin/ls (separate binary) | busybox ls (same binary handles all commands) |
| /bin/cp (separate binary) | busybox cp |
| /bin/grep (separate binary) | busybox grep |
| Total /bin: ~162 commands as separate files | BusyBox /bin: 89 commands — 1 file, symlinked |
| /bin size: several MB | busybox binary: ~1.4 MB total for all commands |
| Cannot customise easily | Remove any command you don't need at compile time |

📝 Note

BusyBox uses symbolic links (symlinks) as a clever trick. The single 'busybox' binary

checks argv[0] (the name it was called with) to decide which command to run.

So 'ls' is just a symlink pointing to busybox. When you run 'ls', busybox sees

its name is 'ls' and runs ls functionality. This saves enormous amounts of storage.

## 10.2 Compiling BusyBox

Download from: https://busybox.net/downloads/

Step 1: Apply default configuration

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- defconfig

Step 2: Customize (optional) — enable static build

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- menuconfig

Navigate to: BusyBox Settings → Build BusyBox as a static binary (no shared libs)

Select this option and save.

Why static? Embedded boards may not have the standard C library (glibc) installed.

A statically linked BusyBox has everything built in and runs anywhere.

Step 3: Compile and install to your rootfs directory

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- CONFIG_PREFIX=<install_path> install

Replace <install_path> with your workspace directory, e.g., /home/user/BBB_RFS

After install, your rootfs will contain:

bin/ → 89 commands (all symlinks to busybox)

sbin/ → 70 system commands (all symlinks to busybox)

usr/ → additional utilities

linuxrc → symlink to busybox (used with initramfs)

Verify: du -sh <install_path> → approximately 1.4 MB total

Step 4: Install kernel modules into rootfs

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- INSTALL_MOD_PATH=<install_path> modules_install

This creates <install_path>/lib/modules/<kernel-version>/kernel/drivers/

Also manually create: mkdir <install_path>/dev (empty device directory)

## 10.3 BusyBox Init System

BusyBox provides its own init program which becomes PID 1 (the first user space process). It reads /etc/inittab and runs scripts from /etc/init.d/ in alphabetical order. Understanding this is crucial for embedded Linux system startup customization.

📷 Fig 10.1 — Linux 'init' Program — two implementations. LEFT branch: BusyBox 'init' — when BusyBox is PID 1, it reads /etc/inittab and then runs the rcS script from /etc/init.d/. rcS is the master startup script. RIGHT branch: System V 'init' — the traditional Linux init used on desktop systems, which reads /etc/inittab for service configuration. Both implementations ultimately look for scripts in /etc/init.d/ (shown at bottom where both arrows converge). For embedded BBB with BusyBox, we use the LEFT branch: BusyBox init → rcS → individual S## scripts.

| Script Name | Purpose |
| --- | --- |
| rcS | Master startup script — mounts /proc, runs all S* scripts in /etc/init.d/ |
| rcK | Master shutdown script — runs all K* scripts to stop services |
| S01Logging | First to run — starts syslog (system log) and klog (kernel log) |
| S02module | Loads kernel modules (e.g., g_ether for USB-over-Ethernet) |
| S04network | Brings up network interfaces (runs 'ifup -a') |
| S05sshd (or S50sshd) | Starts the SSH daemon for remote access |

📝 Note

Script naming convention: S + two-digit number + name

S = Start script (run by rcS during boot)

K = Kill/stop script (run by rcK during shutdown)

Number = execution order (01 runs before 50)

Example: S01Logging runs before S04network which runs before S50sshd

### Required Directories in rootfs:

mkdir -p etc/init.d    # init scripts

mkdir -p proc          # virtual filesystem for process info

mkdir -p sys           # virtual filesystem for hardware info

mkdir -p dev           # device files

mkdir -p etc/network   # network configuration

### Loading Kernel Modules at Runtime:

# Manually load a kernel module:

modprobe g_ether

# modprobe is intelligent — it checks dependencies first

# List loaded modules:

lsmod

# After loading g_ether, USB0 network interface appears:

ifconfig usb0 192.168.6.2 up

ifconfig   # usb0 now visible

SECTION 11 — Buildroot

# 11. Buildroot — Automated Embedded Linux Build System

Buildroot is a tool that automates everything we have done manually in the previous sections. Instead of individually downloading and compiling U-Boot, the Linux kernel, BusyBox, and third-party packages, Buildroot does it all with a single 'make' command. It generates a complete, ready-to-use embedded Linux image.

## 11.1 What Buildroot Can Build

- Cross-compilation toolchain (or use an external one like Linaro)

- Bootloader (U-Boot, MLO/SPL)

- Linux kernel (with Device Tree Blobs)

- Root filesystem (using BusyBox, plus optional packages)

- Additional packages: OpenSSH, Python, web servers, databases, and hundreds more

💡 Tip

Think of Buildroot as a 'recipe book' for your embedded Linux system.

You tell it what hardware you have (BBB), what features you want (SSH, specific kernel version),

and it downloads, compiles, and packages everything automatically.

## 11.2 Getting Started with Buildroot

Download from: https://buildroot.org/download.html

| Directory | Contents |
| --- | --- |
| configs/ | Pre-made configurations for popular boards (beaglebone_defconfig is here) |
| board/ | Board-specific files, post-build scripts, and READMEs |
| board/beaglebone/ | BBB-specific overlay files and build scripts |
| board/beaglebone/readme.txt | Step-by-step instructions for building for BBB |
| package/ | Package recipes for hundreds of software packages |
| output/images/ | All compiled output files appear here after build |

## 11.3 Configuring and Building for BBB

Step 1: Load BBB default configuration

cd buildroot-2017.05

make beaglebone_defconfig

Step 2: Open configuration menu

make menuconfig

### Key menuconfig Settings for BBB:

| Menu Path | Setting |
| --- | --- |
| Toolchain → Toolchain Type | Select 'External toolchain' (to use your Linaro installation) |
| Toolchain → Toolchain | Linaro (auto-detected if installed) |
| Toolchain → Toolchain origin | Pre-installed toolchain |
| Toolchain → Toolchain path | Path to your toolchain bin/ directory |
| System configuration → System hostname | Your custom hostname (e.g., my-bbb) |
| System configuration → System banner | Welcome message (e.g., Welcome to Buildroot-BBB) |
| System configuration → Init system | BusyBox |
| System configuration → Root password | Set a root password |
| System configuration → getty (login prompt) | ttyO0 at 115200 baud |
| Linux kernel → Defconfig name | bb.org (or omap2plus for kernel 4.11+) |
| Linux kernel → Kernel binary format | uImage |
| Linux kernel → Load address | 0x80008000 |
| Linux kernel → Device tree source | am335x-boneblack |
| Target packages → Networking → openssh | Enable for SSH access |
| Bootloader → U-Boot | Disable (we already have our own) |

Step 3: Build everything

make -j4

This can take 30–90 minutes on first run (downloads and compiles everything).

Output files in buildroot-2017.05/output/images/:

MLO, u-boot.img, uImage, am335x-boneblack.dtb, rootfs.tar

Buildroot Output Files

📷 Fig 11.1 — Result of the Buildroot build showing output/images/ directory contents. After 'make -j4' completes, these files are generated: am335x-boneblack.dtb (Device Tree for BBB), am335x-bone.dtb, am335x-evm.dtb, am335x-evmsk.dtb (DTBs for other AM335x boards), boot.vfat (FAT boot partition image), MLO (Secondary Program Loader), rootfs.ext2 (root filesystem in ext2 format), rootfs.tar (root filesystem as tarball — use this to extract to NFS), sdcard.img (complete SD card image ready to dd), u-boot.img (U-Boot bootloader), uEnv.txt (auto-generated boot script), zImage (compressed kernel). The sdcard.img is the most convenient — you can dd it directly to an SD card.

## 11.4 Deploying Buildroot Output

# Remove old rootfs from NFS directory

sudo rm -rf /srv/nfs/bbb/*

# Extract new rootfs

sudo tar -xf output/images/rootfs.tar -C /srv/nfs/bbb

# Copy kernel and DTB to TFTP directory

cp output/images/uImage /var/lib/tftpboot/

cp output/images/am335x-boneblack.dtb /var/lib/tftpboot/

# Reboot BBB and it will use the new images

## 11.5 Enabling SSH in Buildroot

After enabling OpenSSH in menuconfig and rebuilding, you may need to configure the sshd config on the target:

# On host PC, edit the rootfs (before extracting) or after SSH fails:

vi /srv/nfs/bbb/etc/ssh/sshd_config

# Uncomment or add:

PermitRootLogin yes

# Restart NFS service:

sudo service nfs-kernel-server restart

# From host PC, SSH into BBB:

ssh -l root 192.168.7.2

# Enter root password when prompted

# From BBB, SSH into host PC:

ssh kiran@192.168.7.1

SECTION 12 — Linux Source Tree & Boot Flow

# 12. Linux Source Tree & Detailed Boot Flow

Understanding the Linux source tree structure and the detailed boot flow is essential for any embedded Linux developer. This section maps the boot sequence to exact source files.

## 12.1 Linux Kernel Source Tree — Key Directories

| Directory/File | Contents & Purpose |
| --- | --- |
| arch/ | Architecture-specific code (arm, x86, mips, riscv, arm64, etc.) |
| arch/arm/boot/ | ARM boot files: head.S, compressed/, dts/ |
| arch/arm/boot/compressed/head.S | FIRST file executed after U-Boot hands over control. Decompresses kernel. |
| arch/arm/boot/compressed/misc.c | Kernel decompression logic — unzips the compressed kernel image |
| arch/arm/kernel/head.S | SECOND head.S — CPU init, MMU setup, calls start_kernel() |
| arch/arm/kernel/head-common.S | Calls the C main function after assembly setup completes |
| arch/arm/lib/bootm.c | U-Boot file: implements boot_jump_linux() to call kernel entry |
| arch/arm/mm/ | ARM memory management (page tables, cache, TLB) |
| arch/arm/boot/dts/ | All Device Tree source files for ARM boards |
| init/main.c | Architecture-independent kernel main — start_kernel() is here |
| kernel/ | Core kernel: scheduler, timers, signals, modules |
| mm/ | Memory management subsystem |
| drivers/ | All hardware device drivers (tens of thousands of files) |
| fs/ | Filesystem implementations (ext4, fat, nfs, proc, sys, etc.) |
| net/ | Network protocols (TCP/IP, UDP, NFS, etc.) |
| include/ | Kernel header files |
| Documentation/ | Extensive kernel documentation |
| Makefile | Top-level build system |
| Kconfig | Configuration system (what menuconfig reads) |
| .config | Generated after menuconfig — contains all kernel settings |

## 12.2 Detailed Boot Flow: U-Boot to Linux User Space

📷 Fig 12.1 — Control Flow during Linux Boot (source file level diagram). U-Boot (top-left) calls into Linux's Boot Strap Loader layer (step 1): arch/arm/boot/compressed/head.S runs first, which calls misc.c (step 2) to decompress the kernel, then head.S regains control. Step 3: the second head.S at arch/arm/kernel/head.S runs (CPU/MMU initialisation). Step 4: head-common.S bridges to C code. Step 5: init/main.c start_kernel() function is called. Step 6: start_kernel() eventually launches the init process (PID 1) in User Applications layer.

| # | Source File | What Happens |
| --- | --- | --- |
| 1 | arch/arm/lib/bootm.c (U-Boot) | boot_jump_linux() creates kernel_entry function pointer. Calls kernel_entry(0, machid, r2). r2 = DTB address in DDR RAM. |
| 2 | arch/arm/boot/compressed/head.S | First kernel assembly code runs. Sets up basic environment, prepares for decompression. |
| 3 | arch/arm/boot/compressed/misc.c | Decompresses the kernel image from its compressed format to plain binary in RAM. |
| 4 | arch/arm/kernel/head.S | CPU-specific initialisation, validates processor architecture, sets up page tables, enables MMU. |
| 5 | arch/arm/kernel/head-common.S | Final setup, then calls the C function start_kernel(). |
| 6 | init/main.c: start_kernel() | Arch-independent kernel init: memory, interrupts, scheduler, filesystems, devices all initialised. |
| 7 | init/main.c: rest_init() | Creates two kernel threads: kernel_init (PID 1 candidate) and kthreadd (PID 2, kernel thread manager). |
| 8 | kernel_init thread | Frees __init memory. Tries to exec /sbin/init → /etc/init → /bin/init → /bin/sh in order. |
| 9 | /sbin/init or busybox init | PID 1 starts. Reads /etc/inittab, runs /etc/init.d/rcS, starts all services. |

## 12.3 kernel_init — Finding PID 1

The kernel_init thread tries to find and run the first userspace program in this exact order:

/* From init/main.c */

if (execute_command) {

run_init_process(execute_command);  /* init= kernel argument */

}

run_init_process("/sbin/init");

run_init_process("/etc/init");

run_init_process("/bin/init");

run_init_process("/bin/sh");

/* If ALL fail: */

panic("No init found. Try passing init= option to kernel");

📝 Note

You can specify a custom init program by passing 'init=<path>' in the kernel bootargs.

Example: setenv bootargs ... init=/bin/myapp

This is useful in embedded systems where the default /sbin/init is not needed.