ZEPHYR RTOS

Embedded RTOS / zephyr RTOS • 03-06-2026 • Aravinth K

Bare Metal vs Free RTOS vs Zephyr

| Feature | Meaning |
| --- | --- |
| Drivers | You read the datasheet and write every register yourself |
| Build system | You choose — Makefile, IDE, whatever you want |
| Device Tree | Doesn't exist. You hardcode pin numbers, addresses |
| Subsystems | You implement USB/BLE yourself or use vendor library |
| Portability | If you switch from STM32 → NRF, you rewrite everything |

Bare Metal — What You Already Know

- We will write everything

// YOU write this GPIO driver from scratch // reading datasheet, writing register addresses manually *((volatile uint32_t *)0x40020000) = 0x01; // raw register write

FreeRTOS — One Step Up

- FreeRTOS gives you only the kernel (threads, semaphores, queues). That's it. Everything else — drivers, BLE, USB — you still bring yourself from vendor SDKs.

Zephyr — The Full Package

- Zephyr gives you everything in one place, standardized.

- Drivers

|  | Reality |
| --- | --- |
| Bare metal | You write UART driver for STM32 by reading its 800-page manual |
| FreeRTOS | You use STM32's HAL library — but it only works on STM32 |
| Zephyr | You call uart_write() — same function works on ANY chip |

- Build system

|  | Reality |
| --- | --- |
| Bare metal | You manually write Makefiles or use Keil/IAR IDE |
| FreeRTOS | You set up CMake yourself per project |
| Zephyr | You type west build — one command handles everything |

- Device Tree

|  | Reality |
| --- | --- |
| Bare metal | You hardcode #define LED_PIN 13 everywhere in your .c file |
| FreeRTOS | Same — no standard way to describe hardware |
| Zephyr | Hardware is described in a separate file (dts), code never has hardcoded pin numbers |

- Subsystems

|  | Reality |
| --- | --- |
| Bare metal | You find a BLE library, USB library — all different styles, no standards |
| FreeRTOS | You plug in vendor stacks — they may conflict, have different APIs |
| Zephyr | USB, BLE, Networking, File System — all built-in, all standardized |

- Portability

|  | Reality |
| --- | --- |
| Bare metal | Your STM32 code is 100% useless on NRF52 — rewrite everything |
| FreeRTOS | The kernel works on 40+ architectures, but drivers still need porting |
| Zephyr | The kernel works on 40+ architectures, but drivers still need porting |

Zephyr SDK and West

Think of building a house:

- Zephyr SDK = your toolbox (hammer, drill, screwdriver — the actual tools)

- West = your project manager (tells you what to build, in what order, fetches materials)

- Your app = the house you're building

Zephyr SDK provides:

Zephyr SDK contains:

├── Cross-compilers        → gcc for ARM, RISC-V, Xtensa, ARC...

├── Debuggers              → GDB configured for embedded targets

├── Flashing tools         → OpenOCD, pyOCD, nrfjprog etc.

└── Binary utilities       → objcopy, size, nm etc.

Without SDK:

Your C code  →  ??? compiler  →  ❌ doesn't know ARM instructions

With SDK:

Your C code  →  arm-zephyr-eabi-gcc  →  ✅ ARM binary ready to flash

West — The Meta-Tool / Project Manager

West is a Python-based command-line tool that does 4 major jobs:

Command to install

pip install west

Job 1 — 📥 Fetch & Manage Source Code

west init myproject # sets up a new Zephyr workspace west update # downloads Zephyr + all dependencies ### Like git clone but smarter — it knows exactly which version of Zephyr and which #modules your project needs. ###

Job 2 — 🏗️ Build Your Application

west build -b nrf52840dk -- app/ # build for a specific board ### Internally calls CMake + Ninja/Make, but YOU don't need to write any CMake commands. West handles it. ###

Job 3 — ⚡ Flash to Hardware

west flash # flash the built binary to your board ### Automatically picks the right flashing tool (JLink, OpenOCD, etc.) based on your board. ###

Job 4 — 🐛 Debug

west debug # starts GDB debug session west attach # attach to already running target

What a West Workspace Looks Like

myworkspace/ ├── .west/ ← west's config lives here │ └── config ├── zephyr/ ← entire Zephyr OS source code │ ├── kernel/ │ ├── drivers/ │ ├── boards/ │ ├── subsys/ │ └── ... ├── modules/ ← external libraries (mbedTLS, LVGL, etc.) │ ├── crypto/ │ ├── fs/ │ └── ... └── your_app/ ← YOUR application code goes here ├── src/ │ └── main.c ├── CMakeLists.txt └── prj.conf

SDK installation steps

Make sure environments are activated

west sdk install arm-zephyr-eabi west sdk install x86-64-zephyr-elf

What is x86-64-zephyr-elf?

- It is a tool chain that compiles Zephyr applications to run on x86-64 architecture — but not your Windows/Linux OS.

- It compiles Zephyr to run on x86-64 hardware bare-metal — meaning:

Normal Linux/Windows app: Your App → OS (Windows/Linux) → Hardware Zephyr on x86-64: Zephyr + Your App → directly on x86-64 Hardware (no OS underneath)

Use Case 1 — QEMU Emulation (Most Common for Beginners)

west build -b qemu_x86 west build -t run # runs Zephyr inside QEMU emulator on your PC

- QEMU is a software emulator that pretends to be a hardware board. Zephyr runs inside it on your PC — no real hardware needed. Perfect for learning and testing.

Use Case 2 — Industrial x86 Embedded Hardware

- Some industrial systems use x86-based embedded boards like:

Intel Edison

Up Board

Some industrial PLCs

Certain gateway devices

These run Zephyr bare-metal on x86 hardware — no Linux, no Windows, just Zephyr directly.

Why a Separate Toolchain Though?

PC already has gcc for x86-64. Why not use that?

Your PC's gcc: Compiles code → links against Linux libraries → produces Linux ELF binary Needs Linux OS to run ✓ x86-64-zephyr-elf gcc: Compiles code → links against Zephyr only → produces bare-metal ELF binary Runs WITHOUT any OS — directly on hardware ✓

- The -elf suffix is the key — it means no OS assumed, pure bare-metal binary.

Zephyr hello world example

C:\Corsair\zephyrproject>west build -p always -b qemu_x86_64 zephyr\samples\hello_world -- west build: making build dir C:\Corsair\zephyrproject\build pristine -- west build: generating a build system Loading Zephyr default modules (Zephyr base). -- Application: C:/Corsair/zephyrproject/zephyr/samples/hello_world -- CMake version: 4.2.3 -- Found Python3: C:/Users/kvw1cob/AppData/Local/Programs/Python/Python311/python.exe (found suitable exact version "3.11.9") found components: Interpreter -- Cache files will be written to: C:/Corsair/p2406/stw/zephyr-p2406/zephyr/.cache -- Zephyr version: 3.3.0 (C:/Corsair/p2406/stw/zephyr-p2406/zephyr) -- Found west (found suitable version "1.5.0", minimum required is "0.7.1") -- Board: qemu_x86_64 -- ZEPHYR_TOOLCHAIN_VARIANT not set, trying to locate Zephyr SDK -- Found host-tools: zephyr 0.15.2 (C:/Corsair/zephyrproject/zephyr-sdk-0.15.2_windows-x86_64/zephyr-sdk-0.15.2) -- Found toolchain: zephyr 0.15.2 (C:/Corsair/zephyrproject/zephyr-sdk-0.15.2_windows-x86_64/zephyr-sdk-0.15.2) -- Found Dtc: C:/ProgramData/chocolatey/bin/dtc.exe (found suitable version "1.6.1", minimum required is "1.4.6") -- Found BOARD.dts: C:/Corsair/p2406/stw/zephyr-p2406/zephyr/boards/x86/qemu_x86/qemu_x86_64.dts -- Generated zephyr.dts: C:/Corsair/zephyrproject/build/zephyr/zephyr.dts -- Generated devicetree_generated.h: C:/Corsair/zephyrproject/build/zephyr/include/generated/devicetree_generated.h -- Including generated dts.cmake file: C:/Corsair/zephyrproject/build/zephyr/dts.cmake C:/Corsair/zephyrproject/build/zephyr/zephyr.dts:37.24-43.4: Warning (interrupt_provider): /ioapic@fec00000: Missing #address-cells in interrupt provider Parsing C:/Corsair/p2406/stw/zephyr-p2406/zephyr/Kconfig Loaded configuration 'C:/Corsair/p2406/stw/zephyr-p2406/zephyr/boards/x86/qemu_x86/qemu_x86_64_defconfig' Merged configuration 'C:/Corsair/zephyrproject/zephyr/samples/hello_world/prj.conf' Configuration saved to 'C:/Corsair/zephyrproject/build/zephyr/.config' Kconfig header saved to 'C:/Corsair/zephyrproject/build/zephyr/include/generated/autoconf.h' -- The C compiler identification is GNU 12.1.0 -- The CXX compiler identification is GNU 12.1.0 -- The ASM compiler identification is GNU -- Found assembler: C:/Corsair/zephyrproject/zephyr-sdk-0.15.2_windows-x86_64/zephyr-sdk-0.15.2/x86_64-zephyr-elf/bin/x86_64-zephyr-elf-gcc.exe CMake Warning at C:/Corsair/p2406/stw/zephyr-p2406/zephyr/subsys/random/CMakeLists.txt:11 (message): Warning: CONFIG_TIMER_RANDOM_GENERATOR is not a truly random generator. This capability is not secure and it is provided for testing purposes only. Use it carefully. -- Configuring done (12.3s) -- Generating done (0.4s) -- Build files have been written to: C:/Corsair/zephyrproject/build -- west build: building application [1/123] Generating include/generated/version.h -- Zephyr version: 3.3.0 (C:/Corsair/p2406/stw/zephyr-p2406/zephyr), build: zephyr-v3.3.0-19-ge69969a20600 [113/123] Linking C executable zephyr\zephyr_pre0.elf; Logical command for additional byproducts on target: zephyr_pre0 [117/123] Linking C executable zephyr\zephyr_pre1.elf; Logical command for additional byproducts on target: zephyr_pre1 [123/123] Linking C executable zephyr\zephyr.elf; Generating files from zephyr.elf for board: qemu_x86_64 Memory region Used Size Region Size %age Used RAM: 112 KB 3 MB 3.65% LOCORE: 32 KB 60 KB 53.33% C:\Corsair\zephyrproject>

Log Explanation

Stage 1 — West Takes Charge

-- west build: making build dir C:\Corsair\zephyrproject\build pristine -- west build: generating a build system

- -p always means pristine build — wipe the previous build folder and start fresh. Like a clean compile. West creates a fresh build/ folder and starts setting up CMake inside it.

Stage 2 — CMake Finds Everything It Needs

-- Application: C:/Corsair/zephyrproject/zephyr/samples/hello_world -- CMake version: 4.2.3 -- Found Python3: ... python.exe (version 3.11.9) -- Zephyr version: 3.3.0 -- Found west (version 1.5.0) -- Board: qemu_x86_64

CMake is doing a checklist here:

Found the app I need to build  → hello_world

CMake itself is present        → 4.2.3

Python is available            → 3.11.9 (needed for build scripts)

Zephyr source is found         → version 3.3.0

West is available              → 1.5.0

Target board is identified     → qemu_x86_64

Stage 3 — SDK & Toolchain Detection

-- ZEPHYR_TOOLCHAIN_VARIANT not set, trying to locate Zephyr SDK -- Found host-tools: zephyr 0.15.2 (C:/Corsair/zephyrproject/...) -- Found toolchain: zephyr 0.15.2 (C:/Corsair/zephyrproject/...)

You didn't manually tell West where the SDK is. So it auto searched and found it.

ZEPHYR_TOOLCHAIN_VARIANT not set ↓ West searches known locations on your PC ↓ Found SDK 0.15.2 at C:\Corsair\zephyrproject\zephyr-sdk-0.15.2... ↓ Confirmed: x86_64-zephyr-elf compiler will be used for this build

Stage 4 — Device Tree Processing

-- Found Dtc: C:/ProgramData/chocolatey/bin/dtc.exe (version 1.6.1) -- Found BOARD.dts: .../boards/x86/qemu_x86/qemu_x86_64.dts -- Generated zephyr.dts: C:/Corsair/zephyrproject/build/zephyr/zephyr.dts -- Generated devicetree_generated.h: .../include/generated/devicetree_generated.h

This is Device Tree compilation — a 3 step process:

Step 1: DTC tool is found (Device Tree Compiler) ↓ Step 2: Board's .dts file is read qemu_x86_64.dts ← describes the virtual hardware of QEMU (what peripherals exist, memory map, interrupt lines etc.) ↓ Step 3: Final merged zephyr.dts is generated in build folder ↓ Step 4: devicetree_generated.h is created ← this is a C header file your code uses to access hardware

Stage 5 — Kconfig Processing

Parsing C:/Corsair/.../zephyr/Kconfig Loaded configuration '...qemu_x86_64_defconfig' Merged configuration '.../hello_world/prj.conf' Configuration saved to '.../build/zephyr/.config' Kconfig header saved to '.../include/generated/autoconf.h'

This is feature selection — deciding what goes into your binary:

qemu_x86_64_defconfig ← board's default settings + prj.conf (hello_world) ← your app's specific settings ↓ Merged into final .config ← master config for this build ↓ autoconf.h is generated ← C header with all #define CONFIG_xxx macros

Stage 6 — Actual Compilation

-- The C compiler identification is GNU 12.1.0 -- The CXX compiler identification is GNU 12.1.0 -- Found assembler: .../x86_64-zephyr-elf/bin/x86_64-zephyr-elf-gcc.exe

Compilers are confirmed:

C code → x86_64-zephyr-elf-gcc (GNU 12.1.0) C++ code → x86_64-zephyr-elf-g++ (GNU 12.1.0) ASM code → x86_64-zephyr-elf-gcc (same, handles assembly too)

Stage 7 — The 123 Build Steps

[1/123] Generating include/generated/version.h [113/123] Linking C executable zephyr\zephyr_pre0.elf [117/123] Linking C executable zephyr\zephyr_pre1.elf [123/123] Linking C executable zephyr\zephyr.elf

This is the actual compilation happening. Let me explain the 3 ELF files:

zephyr_pre0.elf → First pass link Used to calculate actual sizes of sections (how big is code? how big is data?) ↓ zephyr_pre1.elf → Second pass link Fine-tunes memory layout using sizes from pre0 ↓ zephyr.elf → FINAL binary ✅ This is what gets flashed / run in QEMU

Why 3 passes?

- Because Zephyr needs to know exact memory sizes to set up the memory protection, linker script, and stack sizes correctly before producing the final binary.

Stage 8 — Memory Report

Memory region Used Size Region Size %age Used RAM: 112 KB 3 MB 3.65% LOCORE: 32 KB 60 KB 53.33%

This is your memory usage report — extremely important in embedded:

RAM: ├── Total available → 3 MB (QEMU virtual RAM) ├── Used → 112 KB └── Usage → 3.65% ✅ very healthy LOCORE: (Low Core — early boot code region) ├── Total available → 60 KB ├── Used → 32 KB └── Usage → 53.33% ✅ acceptable

- On a real embedded board (like NRF52840 with 256KB RAM), this report is critical. If RAM hits 100%, your app won't boot. Always watch this number.

Complete Build Flow Summary

west build command ↓ CMake — finds app, board, SDK, Python ↓ DTC — compiles Device Tree → devicetree_generated.h ↓ Kconfig — merges configs → autoconf.h ↓ GCC — compiles all 123 source files ↓ Linker — pre0 → pre1 → final zephyr.elf ↓ Memory report printed ↓ ✅ Build complete — ready to run!

Testing with Real Hardware — LED Blinky

Step 1 — Check If Your Board Is Supported

The west boards command lists all boards supported by Zephyr. However, just because a board appears in this list does not mean it supports every sample application. Each sample has its own hardware requirements.

- west boards

Step 2 — Check the Sample's Hardware Requirements

Every Zephyr sample has a README.rst file that lists what the board must support. For the blinky example, open:

zephyr\samples\basic\blinky\README.rst

You will find a requirement like this:

- Your board must: - 1. Have an LED connected via a GPIO pin - (these are called "User LEDs" on many of Zephyr's boards). - 2. Have the LED configured using the "led0" devicetree alias.

This means your board's .dts file must define a led0 alias pointing to a real GPIO-connected LED. If this alias is missing, the blinky sample cannot compile for that board.

Step 3 — Verify the led0 Alias in the Board's DTS File

✅ Supported Board Example — STM32F746G-DISCO

Open the board's device tree file:

zephyr\boards\st\stm32f746g_disco\stm32f746g_disco.dts

You will find:

- aliases { - led0 = &green_led_1; - sw0 = &user_button; - };

The led0 alias exists and points to green_led_1 which is a real GPIO-connected LED on the board. ✅ This board supports the blinky sample.

Unsupported Board Example — QEMU Cortex-M3

Open the board's device tree file:

zephyr\boards\qemu\cortex_m3\qemu_cortex_m3.dts

There is no led0 alias defined anywhere in this file. This is expected — QEMU is a virtual board and has no physical LED hardware. ❌ The blinky sample cannot be used with this board.

Step 4 — Build for STM32F746G-DISCO

Run the following command to build the blinky sample for the STM32F746G-DISCO board:

- west build -p always -b stm32f746g_disco zephyr/samples/basic/blinky

Flag explanations:

- -p always — pristine build, clears previous build output before starting

- -b stm32f746g_disco — specifies the target board

- last argument — path to the sample application

At the end of a successful build you will see a memory usage report like:

- Memory region Used Size Region Size %age Used - FLASH: 8 KB 1 MB 0.78% - RAM: 4 KB 320 KB 1.25%

Step 5 — Connect the Board and Flash

🔌 How to Connect STM32F746G-DISCO

The STM32F746G-DISCO board has two USB ports:

- CN15 — USB ST-Link port ← use THIS one for flashing and debugging - CN13 — USB FS OTG port ← this is for USB application use, NOT for flashing

Always connect the ST-Link USB port (CN15) to your PC when flashing. The ST-Link is an on-board debugger/programmer chip that handles the communication between your PC and the STM32 microcontroller. It is not a plain USB connection.

Flash the Binary

- west flash

What happens internally:

- west flash - ↓ - West checks the board definition for the default flash runner - ↓ - For STM32F746G-DISCO → runner is "openocd" or "stm32cubeprogrammer" - ↓ - Runner opens ST-Link connection to the board - ↓ - Writes zephyr.elf / zephyr.hex into the board's FLASH memory - ↓ - Board resets and starts running the new firmware

Creating an Application in Zephyr

In Zephyr, you can create an application in two main ways

1. Using a reference workspace application

2. Creating an application by hand

1. Using a reference workspace application

Clone the application template using the below link in zephyrproject workspace

git clone https://github.com/zephyrproject-rtos/example-application.git

Bash

- C:\Corsair\zephyrproject\example-application>cd .. - C:\Corsair\zephyrproject>git clone https://github.com/zephyrproject-rtos/example-application.git - Cloning into 'example-application'... - remote: Enumerating objects: 761, done. - remote: Counting objects: 100% (45/45), done. - remote: Compressing objects: 100% (30/30), done. - remote: Total 761 (delta 24), reused 15 (delta 15), pack-reused 716 (from 3) - Receiving objects: 100% (761/761), 153.93 KiB ¦ 213.00 KiB/s, done. - Resolving deltas: 100% (283/283), done. - C:\Corsair\zephyrproject>cd example-application - C:\Corsair\zephyrproject\example-application>ls - CMakeLists.txt Kconfig LICENSE README.md app boards doc drivers dts include lib scripts tests west.yml zephyr - C:\Corsair\zephyrproject\example-application>cp -r app my-app-1 - C:\Corsair\zephyrproject\example-application>cd my-app-1 - C:\Corsair\zephyrproject\example-application\my-app-1>ls - CMakeLists.txt Kconfig VERSION boards debug.conf prj.conf sample.yaml src - C:\Corsair\zephyrproject\example-application\my-app-1>tree /f - Folder PATH listing for volume Windows - Volume serial number is 000000AE 42C4:0F99 - C:. - │ CMakeLists.txt - │ debug.conf - │ Kconfig - │ prj.conf - │ sample.yaml - │ VERSION - │ - ├───boards - │ nucleo_f302r8.overlay - │ - └───src - main.c - C:\Corsair\zephyrproject\example-application\my-app-1>

CMakeLists.txt

- Purpose: Entry point for the build system. Tell CMake where everything is.

- cmake_minimum_required(VERSION 3.20.0) - find_package(Zephyr REQUIRED HINTS $ENV{ZEPHYR_BASE}) - project(my-app-1) - target_sources(app PRIVATE src/main.c)

cmake_minimum_required(VERSION 3.20.0)

- Minimum CMake version needed. Build fails if CMake is older.

find_package(Zephyr ...)

- THIS IS THE MOST IMPORTANT LINE

- Loads the entire Zephyr build system into your project

- Without this, CMake has no idea about Zephyr, boards, DTS, Kconfig

- ZEPHYR_BASE env variable points to where Zephyr source is installed

project(my-app-1)

- Names your project — appears in build logs

target_sources(app PRIVATE src/main.c)

- Tells CMake: "compile src/main.c as part of the app"

- Add more .c files here as your project grows

- "app" is a special CMake target name reserved by Zephyr

Where to Put the .h File

Recommended structure:

- my-app-1/ - ├── CMakeLists.txt - ├── prj.conf - └── src/ - ├── main.c - ├── mymodule.c - └── include/ - └── mymodule.h ← your header file goes here

How to Tell CMake About It

CMakeLists.txt:

- cmake_minimum_required(VERSION 3.20.0) - find_package(Zephyr REQUIRED HINTS $ENV{ZEPHYR_BASE}) - project(myapp) - # Add your header search path - target_include_directories(app PRIVATE src/include) - # Add your source files - target_sources(app PRIVATE - src/main.c - src/mymodule.c - )

target_include_directories tells the compiler — "when you see #include "mymodule.h", look in the src/include/ folder."

- // main.c - #include "mymodule.h" // ← works because src/include is registered

PRIVATE vs PUBLIC — Quick Note

- target_include_directories(app PRIVATE src/include) - # ↑ - # use PRIVATE always for app-level headers - # PUBLIC is only for libraries used by others

prj.conf

Purpose: Default Kconfig configuration — enables/disables Zephyr features for your app.

Example content

- CONFIG_SERIAL=y - CONFIG_LOG=y - CONFIG_GPIO=y - CONFIG_I2C=y

How it works:

- prj.conf (your app settings) - + - boards/stm32f746g_disco_defconfig (board default settings) - ↓ - Merged into final build/.config - ↓ - autoconf.h generated with all #defines - ↓ - Compiler includes/excludes code based on these defines

It merges the changes with default configurations

The Merge Rules

Rule 1 — If only board defconfig has it → kept as is

- board defconfig: (nothing about GPIO) - prj.conf: CONFIG_GPIO=y - result: CONFIG_GPIO=y

Rule 2 — If only prj.conf has it → added

- board defconfig: (nothing about GPIO) - prj.conf: CONFIG_GPIO=y - result: CONFIG_GPIO=y

Rule 3 — If both have it → prj.conf WINS

- board defconfig: CONFIG_LOG_DEFAULT_LEVEL=1 - prj.conf: CONFIG_LOG_DEFAULT_LEVEL=3 - result: CONFIG_LOG_DEFAULT_LEVEL=3 ✅ your value wins

- Board defconfig  =  factory default settings of the board

- prj.conf         =  your customizations on top of it

debug.conf

Purpose: Additional config options that are only applied during debug builds. Keeps your production binary clean and small.

Typical contents:

- CONFIG_DEBUG=y - CONFIG_LOG_DEFAULT_LEVEL=4 # verbose logging - CONFIG_ASSERT=y # enable runtime assertions - CONFIG_THREAD_ANALYZER=y # monitor thread stack usage - CONFIG_SHELL=y # enable debug shell over UART

How to use it:

- # Normal build — uses only prj.conf - west build -b stm32f746g_disco - # Debug build — uses prj.conf + debug.conf merged together - west build -b stm32f746g_disco -- -DEXTRA_CONF_FILE=debug.conf

You never want CONFIG_LOG_DEFAULT_LEVEL=4 in production — it floods UART and wastes flash. debug.conf lets you keep these settings separate and optional.

Kconfig

Purpose: Defines your own custom configuration options that can be set in .conf files — just like Zephyr's built-in CONFIG_* options.

Example contents:

- mainmenu "My Application Config" - config MY_APP_BLINK_INTERVAL_MS - int "LED blink interval in milliseconds" - default 500 - help - Controls how fast the LED blinks. - config MY_APP_ENABLE_SENSOR - bool "Enable sensor reading" - default y - help - Enable or disable the sensor subsystem.

Now in your prj.conf you can write:

- CONFIG_MY_APP_BLINK_INTERVAL_MS=1000 - CONFIG_MY_APP_ENABLE_SENSOR=n

And in your main.c:

- #ifdef CONFIG_MY_APP_ENABLE_SENSOR - sensor_init(); - #endif - k_msleep(CONFIG_MY_APP_BLINK_INTERVAL_MS);

This is how you make your app configurable without changing source code. Very powerful for products that ship in multiple variants.

VERSION

Purpose: Defines your application's version number in a structured way.

Contents:

- VERSION_MAJOR = 1 - VERSION_MINOR = 0 - PATCHLEVEL = 0 - VERSION_TWEAK = 0 - EXTRAVERSION =

This gets automatically converted by the build system into:

- // available in your code as: - #define APP_VERSION_MAJOR 1 - #define APP_VERSION_MINOR 0 - #define APP_VERSION_STRING "1.0.0"

Use it in your app:

- #include <app_version.h> - printk("Firmware version: %s\n", APP_VERSION_STRING);

Critical for production firmware. When a device in the field reports a bug, you need to know exactly which version is running on it.

sample.yaml

Purpose: Metadata file used by Zephyr's test runner (Twister) and CI/CD systems. Describe what this app is and which boards it supports.

Example contents:

- sample: - name: My Application 1 - description: Example Zephyr application - tests: - sample.my_app_1: - tags: led gpio - depends_on: gpio - platform_allow: - - stm32f746g_disco - - nucleo_f302r8 - min_ram: 16 - min_flash: 32

Field explanations:

- name/description → Human readable app info - tags → categorizes the test (gpio, led, uart etc.) - depends_on → board MUST have this hardware capability - platform_allow → only these boards are valid for this app - min_ram/min_flash → board must have at least this much memory

boards/nucleo_f302r8.overlay

Purpose: Board-specific Device Tree customization for your app. Adds or overrides hardware descriptions that the base board .dts file doesn't have or needs changing.

- / { - aliases { - led0 = &myled; - }; - leds { - compatible = "gpio-leds"; - myled: led_0 { - gpios = <&gpioa 5 GPIO_ACTIVE_HIGH>; - label = "My Custom LED"; - }; - }; - }; - &i2c1 { - status = "okay"; - my_sensor: sensor@48 { - compatible = "myvendor,mysensor"; - reg = <0x48>; - }; - };

Why does it exist separately from the board's main .dts:

- zephyr/boards/st/nucleo_f302r8/nucleo_f302r8.dts - → Describes the BOARD (maintained by Zephyr community, you don't touch it) - boards/nucleo_f302r8.overlay - → Describes YOUR APP's hardware additions on top of that board - → You own this file completely

The naming nucleo_f302r8.overlay must exactly match the board name used in west build -b nucleo_f302r8. Zephyr automatically picks it up — no manual wiring needed.

src/main.c

Purpose: Your actual application code. The entry point of your firmware.

Minimal Zephyr app structure:

- #include <zephyr/kernel.h> - #include <zephyr/drivers/gpio.h> - int main(void) - { - printk("My App %s starting...\n", APP_VERSION_STRING); - while (1) { - // your application logic here - k_msleep(1000); - } - return 0; - }

Unlike bare-metal where you write int main() and manage everything, in Zephyr main() runs as a thread — the kernel is already running before main() is even called. The scheduler, timers, and system clock are all live by the time your first line executes.

Summary — Every File's Role

- my-app-1/ - │ - ├── CMakeLists.txt → "How to build this app" - ├── prj.conf → "What features to include (always)" - ├── debug.conf → "What extra features during debugging" - ├── Kconfig → "What options MY app exposes" - ├── VERSION → "What version this firmware is" - ├── sample.yaml → "What boards/tests this app supports" - │ - ├── boards/ - │ └── nucleo_f302r8.overlay → "How hardware looks for THIS board" - │ - └── src/ - └── main.c → "What the app actually does"

What is libapp.a

- It is the compiled form of your application code — an intermediate file generated during the build process.

How It Gets Created

- src/main.c - src/mymodule.c → compiled by GCC → main.c.o - src/sensors.c mymodule.c.o - sensors.c.o - ↓ - ar tool archives them - ↓ - libapp.a

Every .c file in your app gets compiled into a .o (object file), then the ar tool bundles all those .o files into one libapp.a.

Where It Lives

- build/ - └── zephyr/ - ├── libapp.a ← your app code archived here - ├── libzephyr.a ← zephyr kernel archived here - └── zephyr.elf ← final binary (libapp.a + libzephyr.a linked)

The Full Picture

- Your .c files → libapp.a ──┐ - ├── Linker → zephyr.elf → flash to board - Zephyr kernel → libzephyr.a ─┘

What is Menuconfig

- It is a visual interactive configuration interface that lets you browse and change all CONFIG_* options without manually editing .conf files.

- prj.conf          =  editing settings by typing in a text file

- menuconfig        =  browsing settings in an interactive menu UI

How to Launch It

- west build -b stm32f746g_disco # build first (at least once) - west build -t menuconfig # then launch menuconfig

Basic Controls

| Key | Action |
| --- | --- |
| Arrow keys | Navigate up/down |
| Enter | Open submenu |
| Space or Y | Enable option [*] |
| N | Disable option [ ] |
| M | Set as module |
| / | Search for a config option |
| ? | Show help for selected option |
| S | Save changes |
| Q | Quit |

Most Useful Feature — Search

Press / and type anything:

What Happens When You Save

- You change options in menuconfig - ↓ - Press S to save - ↓ - build/zephyr/.config is updated - ↓ - Next west build picks up the changes - ↓ - autoconf.h is regenerated with new #defines

Important Warning

Changes saved in menuconfig go into build/zephyr/.config which is inside the build folder:

build/zephyr/.config   ← temporary, gets wiped on pristine build!

zephy application on custom board

Device tree

What is device tree?

A data format and language for describing hardware in a platform-independent way. Device Tree describes what hardware exists, how it is wired, and how it is configured — so firmware and operating systems can set up devices without hardcoding board-specific details into application code.

Why Device Tree Matters

- In bare-metal coding for STM32, you write register-specific code directly in your application — hardcoded addresses, pin numbers, peripheral configs. This code cannot be reused on any other microcontroller (for example LPC or NRF52) because the registers and addresses are completely different.

- Device Tree solves this by separating board-specific hardware description from application logic:

Without Device Tree: application code contains hardcoded pin numbers, register addresses, peripheral configs → changing the board = rewriting the application With Device Tree: board hardware is described in .dts files application code asks "give me the LED device" → changing the board = only changing the .dts file application code stays exactly the same

Where to Find the DTS File

For the STM32F746G-DISCO board, the board DTS file is located at:

zephyr/boards/st/stm32f746g_disco/stm32f746g_disco.dts

DTS File Structure

Root Node

Everything in a Device Tree lives inside the root node /:

/ { /* all hardware description lives here */ };

The / is the root node — the top of the tree. All other nodes are children or descendants of it

Parent and Child Nodes

Device Tree is organized as a tree of nodes — exactly like a folder structure on your PC:

/ { ← root node leds { ← parent node (container) compatible = "gpio-leds"; green_led_1: led_1 { ← child node gpios = <&gpioi 1 GPIO_ACTIVE_HIGH>; label = "User LD1"; }; }; };

root / └── leds ← parent / container node └── led_1 ← child node

- leds is the parent node — a logical container that groups related hardware

- green_led_1 is the child node — describes one specific LED

- One parent can have many child nodes (multiple LEDs, multiple UARTs, etc.)

Properties

Inside every node, you define properties — key-value pairs that describe the hardware characteristics:

green_led_1: led_1 { gpios = <&gpioi 1 GPIO_ACTIVE_HIGH>; ← property label = "User LD1"; ← property };

| Property | Purpose |
| --- | --- |
| compatible | Tells Zephyr which driver to use for this node |
| gpios | Which GPIO port, pin number, and active state |
| label | Human readable name for the device |
| status | Whether the device is active ("okay") or disabled ("disabled") |
| reg | Memory address or device bus address |

Node Labels

Notice green_led_1: before led_1 — this is a node label:

green_led_1: led_1 { ← "green_led_1" is the label, "led_1" is the node name ... };

Labels let you reference this node from elsewhere in the DTS using &:

aliases { led0 = &green_led_1; ← referencing the node by its label };

Labels are like variable names for nodes. Without a label, you cannot reference a node from another part of the DTS.

Aliases Node

The aliases node is a special node at the root level that gives short, standardized names to hardware:

/ { aliases { led0 = &green_led_1; ← "led0" is the standard name apps use sw0 = &user_button; }; };

This is why the blinky sample works on any board — it always looks for led0 alias. The alias points to whatever the actual LED node is on that specific board. Your application code never needs to know the real node name.

Reopening a Node with &

You can reopen an already defined node anywhere in the DTS to modify or extend it:

&usart1 { pinctrl-0 = <&usart1_tx_pa9 &usart1_rx_pb7>; pinctrl-names = "default"; current-speed = <115200>; status = "okay"; };

- &usart1 — reopen the node that was already defined in an included .dtsi file

- status = "okay" — activate this peripheral (disabled by default in most SoC files)

- current-speed = <115200> — configure baud rate

- pinctrl-0 — assign physical pins to this peripheral

Most peripherals are defined in SoC-level .dtsi files with status = "disabled". You reopen them in your board .dts or app .overlay and set status = "okay" to activate them. This is the standard pattern across all Zephyr boards.

The compatible Property — Most Important Property

The compatible property is the most critical property in any node:

leds { compatible = "gpio-leds"; ← this string selects the driver };

Zephyr uses this string to match the node to its driver in the kernel:

DTS node compatible = "gpio-leds" ↓ Zephyr searches all registered drivers ↓ Finds driver that declares DT_COMPAT_DEFINE(gpio_leds, "gpio-leds") ↓ That driver is bound to this node automatically

Without compatible, Zephyr doesn't know which driver to load for the hardware. It is the bridge between hardware description and software driver.

DTS File Hierarchy — How Includes Work

A board's final DTS is not one file — it is assembled from multiple files included in a hierarchy:

/* stm32f746g_disco.dts */ #include <st/f7/stm32f746Xg.dtsi> ← SoC level (all STM32F746 chips) #include <st/f7/stm32f746nghx-pinctrl.dtsi> ← Pin definitions for this package #include "arduino_r3_connector.dtsi" ← Arduino header pin mapping #include <zephyr/dt-bindings/input/input-event-codes.h> #include <zephyr/dt-bindings/memory-attr/memory-attr.h> #include <zephyr/dt-bindings/memory-attr/memory-attr-arm.h>

The hierarchy from bottom to top:

ARM architecture base (arm.dtsi) ↓ SoC family level (stm32f7.dtsi) ← all STM32F7 chips ↓ SoC specific level (stm32f746Xg.dtsi) ← this specific STM32F746 chip ↓ Board level (stm32f746g_disco.dts) ← this specific board ↓ App overlay (stm32f746g_disco.overlay) ← your app's customizations ↓ Final merged zephyr.dts ← generated in build/ folder

Each layer adds or overrides what the layer below it defined. Your .overlay file at the top has the final say. This is why you can customize hardware behavior per-application without modifying any Zephyr source files.

.dtsi vs .dts vs .overlay

| Extension | Full Form | Purpose |
| --- | --- | --- |
| .dtsi | Device Tree Source Include | Reusable fragments (SoC, architecture level) |
| .dts | Device Tree Source | Complete board description |
| .overlay | Overlay | App-level customization on top of board .dts |

How Device Tree Reaches Your C Code

The DTS files are never directly used by your C code. They go through a compilation step:

.dts + .overlay files ↓ DTC (Device Tree Compiler) ↓ zephyr.dts (final merged file in build/) ↓ devicetree_generated.h (C header, also in build/) ↓ Your C code uses Zephyr DT macros to access it: const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(DT_ALIAS(led0), gpios);

Your C code never reads .dts files directly. The build system compiles the entire Device Tree into a generated C header, and you access it through Zephyr's DT API macros.

Device Tree File Types and Minimal DTS Layout

Key Device Tree File Types

.dts — Device Tree Source

- The top-level board file. You only create this when adding a brand-new board to Zephyr. It gives a complete description of the board: CPU, memory, buses, and peripherals. It includes the relevant .dtsi files and adds board-level details on top.

.dtsi — Device Tree Source Include

- Reusable fragments that are included by .dts or other .dtsi files. These usually contain SoC-level definitions (e.g., stm32f746.dtsi). They cannot be compiled alone and are meant for sharing common hardware descriptions across boards or chip variants.

.overlay — Application Overlay

- A small patch file that overrides or adds to a base .dts. You use overlays to adjust an existing board description for your specific application or hardware add-on — for example, when you want to add and configure an external sensor or peripheral that is not already defined in the board's default DTS. You never modify the board .dts directly — you always use an overlay.

.h — Device Tree Binding Headers

- Header files included inside .dts and .dtsi files to bring in named constants for GPIO flags, clock IDs, interrupt numbers, and other numeric values. For example:

#include <zephyr/dt-bindings/gpio/gpio.h> /* now you can write GPIO_ACTIVE_HIGH instead of the raw number 0 */

Summary — .dts vs .dtsi

- SoC DTSI files describe the microcontroller itself: which peripherals exist on-chip (UARTs, timers, I2C, SPI, etc.) and their base memory addresses. Everything is defined here with status = "disabled" by default — nothing is active until a board or overlay turns it on.

- Board DTS files are specific to each physical board. They include the SoC DTSI and then add the external components (LEDs, buttons, sensors, connectors) and choose which on-chip peripherals are enabled by setting status = "okay".

- A board DTS does not redefine peripherals from scratch. It reuses the nodes already defined in the SoC DTSI and configures them for that board — pins, baud rate, clock frequency, status, etc.

- Together, the SoC DTSI and the board DTS give a full picture of both the MCU internals and the board-level wiring.

- Your application overlay sits on top of all of this and adds or overrides only what your specific application needs — without touching any Zephyr-maintained files.

armv7-m.dtsi ← CPU core (registers, NVIC, SysTick) ↓ stm32f7.dtsi ← STM32F7 family (all F7 chips share this) ↓ stm32f746Xg.dtsi ← this specific STM32F746 chip (flash size, RAM) ↓ stm32f746g_disco.dts ← this specific board (LEDs, buttons, connectors) ↓ stm32f746g_disco.overlay ← your app's customizations ↓ zephyr.dts (final merged output generated in build/)

Minimal DTS File Layout

/dts-v1/; /* Mandatory: tells the DTC compiler this is DTS version 1 format. Must always be the very first line of every .dts file. */ /*=== INCLUDES ===*/ #include <arm/armv7-m.dtsi> /* Mandatory for ARM boards: brings in CPU core node, NVIC (interrupt controller), and SysTick timer definitions */ #include <st/f7/stm32f746Xg.dtsi> /* Mandatory: brings in all on-chip peripheral nodes for this SoC (UART, SPI, I2C, timers, GPIO ports etc.) all with status = "disabled" */ #include <zephyr/dt-bindings/gpio/gpio.h> /* Optional: brings in named constants like GPIO_ACTIVE_HIGH, GPIO_PULL_UP so you don't have to write raw numbers in property values */ /memreserve/ 0x20000000 0x1000; /* Optional: reserves a region of RAM so the kernel never allocates it. Used for shared memory, bootloader communication areas, etc. Format: /memreserve/ <start_address> <size_in_bytes>; */ /*=== ROOT NODE ===*/ / { /* The root node / is mandatory. It is the top of the entire tree. Every other node in the system is a child or descendant of this node. */ compatible = "st,stm32f746g-disco"; /* Mandatory: identifies this board to Zephyr. Format is always "vendor,board-name" in lowercase with hyphens. */ model = "STMicroelectronics STM32F746G-DISCO board"; /* Mandatory: human-readable board name shown in logs and documentation */ /*=== CHOSEN NODE ===*/ chosen { /* Special node that tells Zephyr which devices to use for system-level functions. Common entries: */ zephyr,console = &usart1; /* which UART to use for printk() output */ zephyr,shell-uart = &usart1; /* which UART to use for the Zephyr shell */ zephyr,sram = &sram0; /* which memory region is main RAM */ zephyr,flash = &flash0; /* which memory region is flash */ }; /*=== MEMORY NODES ===*/ sram0: memory@20000000 { /* Describes the main SRAM region. @20000000 is the base address — must match the reg property. */ compatible = "mmio-sram"; reg = <0x20000000 0x50000>; /* reg format: <base_address size_in_bytes> 0x50000 = 320KB which is the RAM size of STM32F746 */ }; /*=== BOARD-LEVEL DEVICES ===*/ leds { /* Container node grouping all on-board LEDs. This node has no address because LEDs are not memory-mapped devices. */ compatible = "gpio-leds"; /* This compatible string binds this node to Zephyr's gpio-leds driver */ green_led_1: led_1 { /* green_led_1 is the node LABEL — used to reference this node elsewhere led_1 is the node NAME */ gpios = <&gpioi 1 GPIO_ACTIVE_HIGH>; /* Format: <&gpio_port pin_number flags> &gpioi = reference to GPIO port I node (defined in SoC DTSI) 1 = pin number 1 GPIO_ACTIVE_HIGH = LED is on when pin is high */ label = "User LD1"; }; }; buttons { /* Container node grouping all on-board buttons */ compatible = "gpio-keys"; /* This compatible string binds this node to Zephyr's gpio-keys driver */ user_button: button_0 { gpios = <&gpioi 11 (GPIO_ACTIVE_LOW ¦ GPIO_PULL_UP)>; /* Button is active when pin goes LOW (button press pulls it to GND) GPIO_PULL_UP enables internal pull-up resistor */ label = "User Button"; }; }; /*=== ALIASES NODE ===*/ aliases { /* Special node that gives standardized short names to nodes. Zephyr sample applications use these standard names so the same app code works on any board. */ led0 = &green_led_1; /* blinky sample looks for "led0" alias — it finds green_led_1 here */ sw0 = &user_button; /* button sample looks for "sw0" alias */ }; }; /*=== PERIPHERAL CONFIGURATION ===*/ &usart1 { /* Reopen the usart1 node that was defined in the SoC DTSI. The & means "reference to existing node" — not creating a new one. */ pinctrl-0 = <&usart1_tx_pa9 &usart1_rx_pb7>; /* Assigns physical pins: TX on PA9, RX on PB7 */ pinctrl-names = "default"; current-speed = <115200>; /* Baud rate */ status = "okay"; /* Activates this peripheral. Without this line it remains disabled. */ }; &gpioi { status = "okay"; /* GPIO port I must also be activated since our LED uses it */ };

Where to Find SoC DTSI Files

Board-specific and SoC-specific DTSI files are located under:

zephyr/dts/arm/ ← all ARM SoC DTSI files organized by vendor

For STM32F3 family as an example:

zephyr/dts/arm/st/f3/ ├── stm32f3.dtsi ← common base for all STM32F3 chips ├── stm32f302.dtsi ← STM32F302 subfamily ├── stm32f302x8.dtsi ← specific flash/RAM variant ├── stm32f302xc.dtsi ├── stm32f303.dtsi ├── stm32f303x8.dtsi ├── stm32f303xb.dtsi ├── stm32f303xc.dtsi ├── stm32f303xe.dtsi ├── stm32f334.dtsi ├── stm32f334x8.dtsi ├── stm32f373.dtsi └── stm32f373xc.dtsi

Each level in the hierarchy adds more specificity:

stm32f3.dtsi → common to ALL STM32F3 chips ↓ stm32f302.dtsi → specific to STM32F302 subfamily ↓ stm32f302x8.dtsi → specific to the exact flash/RAM variant

Root Node — Detailed

- The root node / acts as the mandatory container for the entire hardware description. All other nodes — CPUs, memory, buses, peripherals, board-specific devices — are direct or indirect children of this root node.

/ (root) ├── chosen ← system device assignments ├── aliases ← standardized short names ├── cpus ← CPU core description (from .dtsi) ├── sram0 ← RAM region ├── flash0 ← Flash region ├── leds ← board LEDs ├── buttons ← board buttons ├── usart1 ← UART peripheral (from SoC .dtsi, configured here) ├── i2c1 ← I2C peripheral ├── spi1 ← SPI peripheral └── gpioa..gpiok ← GPIO ports (from SoC .dtsi)

Every piece of hardware in your system appears somewhere in this tree. If a peripheral is not in the Device Tree, Zephyr has no knowledge of it and no driver will be loaded for it.

Device Tree — Complete Node Anatomy

The Generic Node Structure

label: node-name@unit-address { /* --- Identification --- */ compatible = "vendor,device"; reg = <base-address size>; status = "okay"; /* --- Example Properties --- */ string-prop = "hello"; /* string */ int-prop = <1234>; /* integer (phandle/cell) */ bool-prop; /* boolean — presence = true */ byte-array-prop = [12 34 ab cd]; /* raw hex bytes */ string-list-prop = "one", "two"; /* list of strings */ /* --- Child Nodes --- */ child0: child@0 { reg = <0>; /* child's address within parent */ }; child1: child@1 { reg = <1>; }; };

Node Name, Label, and Unit Address

Node Name

The node name is the human-readable identifier of the node:

leds { ... } /* simple node — no address needed */ uart0 { ... } i2c1 { ... }

Naming is not arbitrary — Zephyr and the Device Tree specification define standard node names for common hardware types. You should follow these:

| Hardware | Standard Node Name |
| --- | --- |
| Serial / UART | serial, uart |
| I2C controller | i2c |
| SPI controller | spi |
| GPIO controller | gpio |
| Memory | Memory |
| CPU | cpu |
| LED group | leds |
| Button group | buttons |

- Using standard names makes your DTS compatible with Zephyr tooling, documentation generators, and other developers reading your code.

Unit Address — @unit-address

The @unit-address part after the node name is the primary address of that device on its bus or in memory:

uart0@40011000 { ... } /* UART at memory address 0x40011000 */ i2c1@40005400 { ... } /* I2C controller at memory address 0x40005400 */ sensor@68 { ... } /* sensor at I2C address 0x68 */ cpu@0 { ... } /* first CPU, index 0 */ memory@20000000 { ... } /* RAM starting at address 0x20000000 */

Critical rule: the unit address must exactly match the first value in the reg property of that same node.

i2c1@40005400 { ... } /* I2C bus 1 */ i2c2@40005800 { ... } /* I2C bus 2 — same name, different address, no conflict */

When is Unit Address Optional?

- Unit address is required when the node has a reg property (memory-mapped peripherals, I2C devices, SPI devices). It is omitted when the node has no address — logical containers like leds, buttons, aliases, chosen:

leds { /* no @ — LEDs are not memory-mapped devices */ compatible = "gpio-leds"; ... }; uart0@40011000 { /* @ required — UART has a memory address */ reg = <0x40011000 0x400>; };

compatible Property

The compatible property is a string or list of strings that tells Zephyr which driver to bind to this node:

compatible = "st,stm32f4-i2c";

Format is always: "vendor,device" — vendor name, comma, device name. No spaces. Lowercase. Hyphens allowed.

Is It Driver Binding? Yes.

This is exactly what happens:

DTS node: compatible = "st,stm32f4-i2c" ↓ Zephyr searches all registered drivers ↓ Driver source declares: DT_DRV_COMPAT = st_stm32f4_i2c (commas→underscores, hyphens→underscores) ↓ Match found → this driver is bound to this node ↓ Driver's probe/init function is called at boot

Most Specific and Fallback — The List Form

compatible = "st,stm32f4-i2c", "st,stm32-i2c";

This is a priority list — most specific first, most generic last:

"st,stm32f4-i2c" ← most specific — only for STM32F4 family I2C "st,stm32-i2c" ← fallback — generic STM32 I2C driver

Zephyr tries to match from left to right:

Step 1: Search for driver matching "st,stm32f4-i2c" → Found? Use it. Done. → Not found? Move to next. Step 2: Search for driver matching "st,stm32-i2c" → Found? Use it. Done. → Not found? Node has no driver — warning at build time.

This allows hardware evolution. A new STM32F4-specific driver can be written for better performance, while boards without it gracefully fall back to the generic STM32 I2C driver. Your DTS never needs to change.

DT_DRV_COMPAT — The Driver Side of Binding

In the driver's C source file, the driver declares what compatible string it handles:

/* Inside Zephyr's i2c driver source — i2c_stm32.c */ #define DT_DRV_COMPAT st_stm32f4_i2c /* ↑ "st,stm32f4-i2c" with comma→underscore, hyphen→underscore */ /* This macro registers the driver with Zephyr's device model and binds it to all DTS nodes with compatible = "st,stm32f4-i2c" */ DEVICE_DT_INST_DEFINE(...)

The transformation rule:

DTS compatible string → DT_DRV_COMPAT "st,stm32f4-i2c" → st_stm32f4_i2c commas → underscores hyphens → underscores all lowercase

Compatible and YAML Binding Files

Every compatible string must have a corresponding YAML binding file in Zephyr. This YAML file defines:

- What properties are required for this node

- What properties are optional

- What type each property must be

- Human-readable descriptions of each property

YAML binding files are located at:

zephyr/dts/bindings/ ├── gpio/ │ └── gpio-leds.yaml ├── i2c/ │ └── st,stm32f4-i2c.yaml ← named after the compatible string ├── sensor/ │ └── invensense,mpu6050.yaml └── ...

Example — invensense,mpu6050.yaml:

description: MPU6050 6-axis IMU sensor compatible: "invensense,mpu6050" include: [sensor-device.yaml, i2c-device.yaml] properties: int-gpios: type: phandle-array description: interrupt GPIO pin required: false accel-hz: type: int description: accelerometer sample rate in Hz required: false default: 100

This YAML file tells the build system:

Any DTS node with compatible = "invensense,mpu6050" must follow these property rules. If a required property is missing → build error.

How to Find the Correct Vendor and Device Name

Step 1 — Find the vendor prefix from the official vendor list:

zephyr/dts/bindings/vendor-prefixes.txt

# excerpt st STMicroelectronics nxp NXP Semiconductors nordic Nordic Semiconductor ti Texas Instruments invensense InvenSense Inc. bosch Bosch Sensortec GmbH

Step 2 — Find the device name from the binding file name in:

zephyr/dts/bindings/

Example for MPU6050:

# search for mpu6050 in bindings folder ls zephyr/dts/bindings/sensor/ ¦ grep mpu # result: invensense,mpu6050.yaml #↑ ↑ #vendor device

So the compatible string is: "invensense,mpu6050"

Root-Level Compatible

The root node / also has a compatible property:

/ { compatible = "st,stm32f746g-disco"; model = "STMicroelectronics STM32F746G-DISCO board"; };

This is not for driver binding. It serves a different purpose:

Root compatible = board identity ↓ Used by Zephyr build system to identify the board Used in documentation and CI tooling Used by west twister to match boards for testing NOT used to load any driver

.The reg Property

- reg property describes the address and size of the device's memory region on its parent bus:

reg = <base-address size>;

Examples

/* Memory-mapped peripheral on a CPU address bus */ uart0@40011000 { reg = <0x40011000 0x400>; /* ↑ ↑ base addr size = 1KB of register space */ }; /* RAM region */ sram0: memory@20000000 { reg = <0x20000000 0x50000>; /* ↑ ↑ base addr size = 320KB */ };

reg for I2C Devices — Only One Value

For devices on an I2C bus, reg contains only the 7-bit I2C address — no size, because I2C devices are not memory-mapped:

&i2c1 { mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; /* ↑ I2C address only — no size field 0x68 is the 7-bit I2C address of MPU6050 */ status = "okay"; }; };

Why only one value here but two for memory-mapped peripherals? This is controlled by #address-cells and #size-cells — explained in Part 5.

Relation Between Unit Address and reg

They must always match:

mpu6050@68 { /* unit address = 68 */ reg = <0x68>; /* reg first value = 0x68 — must be the same */ };

The unit address is just a human-readable label in the node name. The reg property is the machine-readable version. They must be consistent.

Is reg Always Required?

No. reg is only required when the node represents a device with an address:

Required: ✅ Memory-mapped peripherals (UART, I2C controller, SPI controller) ✅ Memory regions (RAM, Flash) ✅ Devices on a bus (I2C sensor, SPI flash chip) Not required: ❌ Logical container nodes (leds, buttons) ❌ Special nodes (chosen, aliases) ❌ Child nodes that use index instead of address (some GPIO nodes)

status Property

The status property controls whether a node is active or inactive:

status = "okay"; /* device is enabled — driver will be loaded */ status = "disabled"; /* device is disabled — driver will NOT be loaded */

The Standard Pattern in Zephyr

SoC DTSI files define all peripherals as disabled by default:

/* Inside stm32f746Xg.dtsi — you never edit this */ i2c1: i2c@40005400 { compatible = "st,stm32f4-i2c", "st,stm32-i2c"; reg = <0x40005400 0x400>; status = "disabled"; /* off by default */ };

Board DTS or app overlay activates only what is needed:

/* Inside your board .dts or app .overlay */ &i2c1 { status = "okay"; /* turn it on for this board/app */ };

This keeps unused peripherals completely out of the build. If status = "disabled", no driver code for that peripheral is compiled into your binary — saving flash space.

#address-cells and #size-cells

These two properties are defined in a parent node and control the format of reg in all its child nodes:

parent-node { #address-cells = <1>; /* how many 32-bit cells describe an address */ #size-cells = <1>; /* how many 32-bit cells describe a size */ child@40011000 { reg = <0x40011000 0x400>; /* ↑1 cell ↑1 cell → matches #address-cells=1, #size-cells=1 */ }; };

For I2C Bus — Why Only One Value in reg

The I2C bus node defines:

i2c1: i2c@40005400 { #address-cells = <1>; /* I2C address takes 1 cell */ #size-cells = <0>; /* NO size field — I2C devices have no size */ mpu6050@68 { reg = <0x68>; /* only address, no size — matches #size-cells=0 */ }; };

#address-cells = 1 → reg needs 1 value for address #size-cells = 0 → reg needs 0 values for size total reg values = 1 + 0 = just <0x68>

Compare with CPU address bus:

#address-cells = 1 → reg needs 1 value for address #size-cells = 1 → reg needs 1 value for size total reg values = 1 + 1 = <0x40011000 0x400>

Why I2C Devices Go Inside the I2C Node

&i2c1 { status = "okay"; mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; status = "okay"; }; };

The MPU6050 is placed inside the i2c1 node because:

1. It physically lives on the I2C bus ↓ 2. Its address 0x68 is an I2C bus address — meaningless without knowing which bus ↓ 3. Zephyr's driver model needs to know: - which I2C controller to use (i2c1) - what address to talk to (0x68) ↓ 4. By being a child of i2c1, both facts are expressed automatically

When Zephyr boots and initializes the MPU6050 driver, it automatically knows:

- Use the i2c1 controller

- Communicate at address 0x68

You never hardcode any of this in your C application code.

Standard vs User-Defined Properties

Standard Properties

Defined by the Device Tree specification and Zephyr — recognized by all tools:

| Property | Meaning |
| --- | --- |
| compatible | Driver binding identifier |
| reg | Address and size |
| status | Enable or disable |
| #address-cells | Address cell count for children |
| #size-cells | Size cell count for children |
| interrupts | Interrupt line description |
| clocks | Clock source reference |
| label | Human readable name |
| pinctrl-0 | Pin assignment |

User-Defined Properties

Properties specific to a particular device, defined in its YAML binding file:

mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; int-gpios = <&gpioc 4 GPIO_ACTIVE_HIGH>; /* user-defined — in mpu6050 YAML */ accel-hz = <100>; /* user-defined — in mpu6050 YAML */ };

These properties only make sense for this specific device. The YAML binding file defines them, validates them at build time, and makes them accessible to the driver through DT macros.

Property Types Summary

node@addr { string-prop = "hello"; /* string — in double quotes */ int-prop = <1234>; /* integer cell — in angle brackets */ bool-prop; /* boolean — just presence, no value */ byte-array-prop = [12 34 ab cd]; /* raw bytes — in square brackets, hex */ string-list-prop = "one", "two"; /* array of strings — comma separated */ phandle-prop = <&some_node>; /* reference to another node */ phandle-array = <&gpioa 5 0>; /* reference + extra cells */ };

Child Nodes and Their reg

spi0@40013000 { #address-cells = <1>; #size-cells = <0>; /* SPI devices also use 0 for size */ flash_chip: flash@0 { reg = <0>; /* chip select 0 — this device is on CS0 */ compatible = "jedec,spi-nor"; }; display: display@1 { reg = <1>; /* chip select 1 — this device is on CS1 */ compatible = "ilitek,ili9340"; }; };

For SPI, the reg value is the chip select number, not a memory address. For I2C, it is the 7-bit I2C address. For memory bus, it is the base memory address. The meaning of reg depends entirely on what bus the parent node represents.

Complete Example — MPU6050 on I2C1

&i2c1 { /* Reopen i2c1 node defined in SoC DTSI */ status = "okay"; /* Activate i2c1 controller */ pinctrl-0 = <&i2c1_scl_pb6 &i2c1_sda_pb7>; /* Assign SCL to PB6, SDA to PB7 */ pinctrl-names = "default"; clock-frequency = <I2C_BITRATE_STANDARD>; /* 100kHz standard I2C speed */ mpu6050@68 { /* Node name "mpu6050" @ I2C address 0x68 */ compatible = "invensense,mpu6050"; /* Binds this node to the MPU6050 driver */ reg = <0x68>; /* I2C address of MPU6050. Hardware default is 0x68 when AD0 pin is LOW. If AD0 pin is HIGH, address becomes 0x69. */ status = "okay"; /* Activate this sensor */ int-gpios = <&gpioc 4 GPIO_ACTIVE_HIGH>; /* Optional: interrupt pin connected to PC4 */ }; };

In your C application code you then simply write:

const struct device *imu = DEVICE_DT_GET(DT_NODELABEL(mpu6050)); /* Zephyr finds the device, knows it's on i2c1 at address 0x68 — you never write any of that yourself */

Quick Reference Summary

node-name → standard name describing hardware type @unit-address → primary address on parent bus (must match reg) compatible → "vendor,device" → selects the driver reg → actual address+size (memory) or address only (I2C/SPI) status → "okay" activates, "disabled" deactivates #address-cells → how many cells are used for address in child reg #size-cells → how many cells are used for size in child reg (0 for I2C and SPI buses) label: → node label — used to reference with &label aliases → standardized names used by application samples chosen → system-level device assignments (console, flash, sram)

Device Tree — Phandles, Interrupts, and Specifier Spaces

What is a Phandle

- In Device Tree, every node that has a label automatically gets a unique integer ID assigned by the DTC compiler. This integer ID is called a phandle.

i2c1: i2c@40005400 { /* "i2c1" is the label */ compatible = "st,stm32f4-i2c"; reg = <0x40005400 0x400>; };

When DTC compiles this, it internally assigns:

i2c1 node → phandle = 5 (some unique integer, assigned automatically)

You never write or manage phandle numbers yourself. You always use the &label syntax and DTC handles the numbers behind the scenes.

Node Reference with & — Simple Phandle

A phandle reference is how one node points to another node. You use &label syntax:

Simple Example — Node A referencing Node B

/* Node B — defined somewhere in a DTSI file */ gpioa: gpio@40020000 { compatible = "st,stm32-gpio"; reg = <0x40020000 0x400>; #gpio-cells = <2>; /* explained in Part 6 */ gpio-controller; }; /* Node A — references Node B via phandle */ leds { compatible = "gpio-leds"; green_led: led_0 { gpios = <&gpioa 5 GPIO_ACTIVE_HIGH>; /* ↑ &gpioa is a phandle reference to the gpioa node above this tells Zephyr: "this LED is connected to the gpioa controller" */ }; };

What DTC does internally:

&gpioa → looks up label "gpioa" → finds its phandle number (e.g. 3) → writes integer 3 into the compiled binary (DTB) At runtime, Zephyr reads phandle 3 → looks up which node has phandle 3 → finds the gpioa GPIO controller node → uses it to drive the LED pin

The &label syntax is just a human-friendly way to write cross-node references. The compiled output contains just integer IDs. You never see or manage those integers.

Phandle Used for I2C Controller Reference

Here is a real example — the SMBus node referencing the I2C controller node:

/* I2C controller node — defined in SoC DTSI */ i2c1: i2c@40005400 { compatible = "st,stm32f4-i2c", "st,stm32-i2c"; reg = <0x40005400 0x400>; status = "disabled"; }; /* SMBus node — references i2c1 via a simple phandle property */ smbus1: smbus1 { compatible = "st,stm32-smbus"; #address-cells = <1>; #size-cells = <0>; i2c = <&i2c1>; /* ↑ simple phandle — just a reference to i2c1 node no extra cells, just the node reference itself this tells Zephyr: "smbus1 is built on top of i2c1 hardware" */ status = "disabled"; };

The i2c = <&i2c1> property is a simple phandle — just one reference, no extra data cells after it. It says: "this SMBus controller uses i2c1 as its underlying hardware."

The SMBus driver in C reads this at compile time:

#define DT_DRV_COMPAT st_stm32_smbus /* get the i2c1 device that this smbus node points to */ const struct device *i2c_dev = DEVICE_DT_GET(DT_INST_PHANDLE(0, i2c)); /* Zephyr automatically resolves &i2c1 → the actual i2c1 device struct */

Phandle Used for Pin Control

Now look at pinctrl — this uses a phandle-array (phandle + extra data cells):

&i2c1 { pinctrl-0 = <&i2c1_scl_pb6 &i2c1_sda_pb7>; /* ↑ ↑ two phandle references inside one property each points to a pin configuration node */ pinctrl-names = "default"; status = "okay"; mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; status = "okay"; }; };

Where are i2c1_scl_pb6 and i2c1_sda_pb7 defined? In the pinctrl DTSI file.

Where to Find Pinctrl Files

zephyr/dts/arm/st/f7/stm32f746nghx-pinctrl.dtsi ← for STM32F746 zephyr/dts/arm/st/f4/stm32f407xx-pinctrl.dtsi ← for STM32F407 zephyr/dts/arm/nordic/nrf52840_qiaa-pinctrl.dtsi ← for NRF52840

General pattern:

zephyr/dts/arm/<vendor>/<chip-specific>-pinctrl.dtsi

Inside that pinctrl file you will find thousands of pre-defined pin nodes:

/* Inside stm32f746nghx-pinctrl.dtsi */ pinctrl: pin-controller@40020000 { compatible = "st,stm32-pinctrl"; i2c1_scl_pb6: i2c1_scl_pb6 { /* SCL function on PB6 */ pinmux = <STM32_PINMUX('B', 6, AF4)>; /* PB6, Alternate Function 4 = I2C1_SCL */ bias-disable; drive-open-drain; slew-rate = "very-high-speed"; }; i2c1_sda_pb7: i2c1_sda_pb7 { /* SDA function on PB7 */ pinmux = <STM32_PINMUX('B', 7, AF4)>; bias-disable; drive-open-drain; slew-rate = "very-high-speed"; }; };

So when you write pinctrl-0 = <&i2c1_scl_pb6 &i2c1_sda_pb7> you are saying:

For i2c1 default pin state (pinctrl-0): SCL → use the pin configuration defined in i2c1_scl_pb6 node SDA → use the pin configuration defined in i2c1_sda_pb7 node

The pinctrl driver reads these nodes at boot and configures the GPIO alternate functions automatically.

Interrupts and interrupt-parent

Now look at the hardware diagram from your image:

MPU6050 INT pin → connected to → PA0 on the MCU

The MPU6050 signals the MCU through its INT pin. That pin connects to a GPIO pin (PA0) which is managed by the MCU's GPIO/EXTI controller, not by the I2C bus.

How This Is Described in Device Tree

/* The interrupt controller — defined in SoC DTSI */ nvic: interrupt-controller@e000e100 { compatible = "arm,v7m-nvic"; reg = <0xe000e100 0xc00>; interrupt-controller; /* This property declares: "I am an interrupt controller" No value needed — just presence of this property marks the node */ #interrupt-cells = <2>; /* Each interrupt description needs 2 cells: cell 1 = IRQ number cell 2 = priority */ }; /* GPIO controller — also an interrupt controller for EXTI lines */ gpioa: gpio@40020000 { compatible = "st,stm32-gpio"; reg = <0x40020000 0x400>; interrupt-controller; /* gpioa can also be interrupt provider */ #interrupt-cells = <2>; #gpio-cells = <2>; gpio-controller; }; /* MPU6050 node — inside i2c1 */ &i2c1 { mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; status = "okay"; interrupt-parent = <&gpioa>; /* phandle pointing to which interrupt controller handles this interrupt the MPU6050 INT pin is connected to GPIOA, so gpioa is the parent */ interrupts = <0 GPIO_ACTIVE_HIGH>; /* cell 1 = 0 → pin PA0 (pin index 0 within GPIOA) cell 2 = GPIO_ACTIVE_HIGH → triggered on rising edge */ int-gpios = <&gpioa 0 GPIO_ACTIVE_HIGH>; /* phandle-array describing the same interrupt pin from the GPIO perspective — used by the sensor driver */ }; };

What interrupt-parent Does

interrupt-parent = <&gpioa> ↓ tells DTC and Zephyr: "the interrupt signal from this device is routed through the gpioa controller" then: interrupts = <0 GPIO_ACTIVE_HIGH> ↓ "specifically pin 0 of gpioa, active high signal"

If interrupt-parent is not specified on a node, it inherits from its parent node. This inheritance walks up the tree until an interrupt controller is found.

Where Is the Interrupt Controller YAML Binding?

zephyr/dts/bindings/interrupt-controller/arm,v7m-nvic.yaml

# arm,v7m-nvic.yaml description: ARMv7-M NVIC interrupt controller compatible: "arm,v7m-nvic" include: [interrupt-controller.yaml] properties: reg: required: true interrupt-controller: true "#interrupt-cells": const: 2 description: ¦ cell 1 = IRQ number cell 2 = priority level

The #interrupt-cells: const: 2 defines that every interrupts = <...> property on a child of this controller must provide exactly 2 values.

General location pattern for interrupt controller bindings:

zephyr/dts/bindings/interrupt-controller/ ├── arm,v7m-nvic.yaml ← ARM Cortex-M NVIC ├── nordic,nrf-uarte.yaml ├── st,stm32-exti.yaml ← STM32 external interrupt controller └── ...

Specifier Space and #xxx-cells

This is the mechanism that defines how many extra data cells follow a phandle reference.

The Rule

When you write a phandle-array property like:

gpios = <&gpioa 5 GPIO_ACTIVE_HIGH>; /* ↑ ↑ ↑ phandle extra cells (specifier) */

The number of extra cells after &gpioa is determined by #gpio-cells defined inside the gpioa node:

gpioa: gpio@40020000 { compatible = "st,stm32-gpio"; gpio-controller; #gpio-cells = <2>; /* every phandle reference to gpioa must be followed by exactly 2 cells */ };

So <&gpioa 5 GPIO_ACTIVE_HIGH> = &gpioa + 2 cells (5 and GPIO_ACTIVE_HIGH)

The Specifier Space Name Rule

The property name determines the specifier space — which #xxx-cells property to look for in the referenced node:

- Property name: int-gpios - ↑ - "gpios" → specifier space name = "gpio" - → look for #gpio-cells in the referenced node - Property name: pwms - ↑ - "pwms" → specifier space name = "pwm" - → look for #pwm-cells in the referenced node

- The general rule:

property named "foos" → specifier space = "foo" → look for #foo-cells property named "x-gpios" → specifier space = "gpio" → look for #gpio-cells property named "pwms" → specifier space = "pwm" → look for #pwm-cells

- Any property name ending in -gpios or equal to gpios uses the gpio specifier space.

Phandle-Array Deep Dive with Real Examples

Example 1 — GPIO phandle-array

/* gpioa node — the provider */ gpioa: gpio@40020000 { gpio-controller; #gpio-cells = <2>; /* declares: any reference to me needs 2 extra cells */ }; /* Consumer node */ green_led: led_0 { gpios = <&gpioa 5 GPIO_ACTIVE_HIGH>; /* ↑ ↑ ↑ │ │ └── cell 2: flags (active high/low, pull-up etc.) │ └───── cell 1: pin number (pin 5 of GPIOA) └──────────── phandle: reference to gpioa controller node */ };

The GPIO YAML binding defines what each cell means:

# zephyr/dts/bindings/gpio/gpio-controller.yaml "#gpio-cells": const: 2 description: ¦ cell 1 = pin number cell 2 = GPIO flags (GPIO_ACTIVE_HIGH, GPIO_PULL_UP, etc.)

Example 2 — PWM phandle-array

/* PWM controller node */ pwm15: pwm@40014000 { compatible = "st,stm32-pwm"; #pwm-cells = <3>; /* declares: any reference to me needs 3 extra cells */ }; /* Consumer node */ buzzer: buzzer { pwms = <&pwm15 0 100000 PWM_POLARITY_NORMAL>; /* ↑ ↑ ↑ ↑ │ │ │ └── cell 3: polarity flag │ │ └───────── cell 2: period in nanoseconds (100000ns = 10kHz) │ └──────────── cell 1: PWM channel number (channel 0) └─────────────────── phandle: reference to pwm15 controller */ };

Example 3 — MPU6050 with interrupt GPIO (your image scenario)

&i2c1 { pinctrl-0 = <&i2c1_scl_pb6 &i2c1_sda_pb7>; /* phandle-array: two pin config references for default state each phandle has 0 extra cells (the pin config is fully inside the referenced pinctrl node itself) */ pinctrl-names = "default"; status = "okay"; mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; /* I2C address — simple integer, not a phandle */ int-gpios = <&gpioa 0 GPIO_ACTIVE_HIGH>; /* ↑ ↑ ↑ │ │ └── cell 2: flag — interrupt fires on high signal │ └───── cell 1: pin number PA0 └──────────── phandle: reference to gpioa controller property "int-gpios" ends in "-gpios" → specifier space = "gpio" → look for #gpio-cells in &gpioa → #gpio-cells = <2> → so 2 extra cells expected after &gpioa ✅ */ status = "okay"; }; };

Who Decides the Format of These Properties?

Why do we describe a GPIO pin like <&gpioa 5 GPIO_ACTIVE_HIGH>? Who decides this format?

The answer is the YAML binding file of the provider node.

Step 1: You write int-gpios = <&gpioa 0 GPIO_ACTIVE_HIGH> ↓ Step 2: DTC sees "-gpios" suffix → specifier space = "gpio" ↓ Step 3: DTC looks inside &gpioa node for #gpio-cells finds: #gpio-cells = <2> ↓ Step 4: DTC validates: after &gpioa there must be exactly 2 cells 0 = pin number ✅ GPIO_ACTIVE_HIGH = flags ✅ ↓ Step 5: The GPIO controller YAML binding defines what cell 1 and cell 2 mean: zephyr/dts/bindings/gpio/gpio-controller.yaml cell 1 → "pin" cell 2 → "flags" ↓ Step 6: DTC generates devicetree_generated.h with all this encoded

You cannot use &gpioa as a reference in a phandle-array property unless the gpioa node has #gpio-cells defined. Without #gpio-cells, DTC does not know how many extra cells to consume after the phandle and will give a compile error.

How Phandles Help the Driver and Application Developer

All the DTS phandle references get compiled by DTC into devicetree_generated.h inside your build folder:

build/zephyr/include/generated/devicetree_generated.h

You never read this file directly. Instead Zephyr provides clean macros:

Accessing a GPIO phandle-array in C

#include <zephyr/drivers/gpio.h> /* DT_ALIAS(led0) finds the node pointed to by aliases { led0 = ... } GPIO_DT_SPEC_GET reads the gpios phandle-array from that node and builds a struct containing: device pointer, pin number, flags */ static const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(DT_ALIAS(led0), gpios); int main(void) { gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE); gpio_pin_toggle_dt(&led); }

The gpio_dt_spec struct that gets filled:

struct gpio_dt_spec { const struct device *port; /* pointer to gpioa device */ gpio_pin_t pin; /* pin number = 5 */ gpio_dt_flags_t dt_flags; /* GPIO_ACTIVE_HIGH */ };

Accessing the MPU6050 interrupt GPIO in C

static const struct gpio_dt_spec mpu_int = GPIO_DT_SPEC_GET(DT_NODELABEL(mpu6050), int_gpios); /* ↑ /* "int-gpios" becomes "int_gpios" in C (hyphens→underscores) */ /* Now configure the interrupt */ gpio_pin_configure_dt(&mpu_int, GPIO_INPUT); gpio_pin_interrupt_configure_dt(&mpu_int, GPIO_INT_EDGE_RISING);

Accessing the I2C device for MPU6050

/* Get the i2c1 bus device that mpu6050 sits on */ const struct device *i2c_bus = DEVICE_DT_GET(DT_BUS(DT_NODELABEL(mpu6050))); /* DT_BUS() walks up to the parent i2c1 node automatically */ /* Or get the mpu6050 device directly if a full driver exists */ const struct device *imu = DEVICE_DT_GET(DT_NODELABEL(mpu6050));

The key benefit: you never hardcode i2c1, address 0x68, pin PA0, or any hardware detail in your C code. All of that lives in the Device Tree. If the hardware changes, you update the DTS/overlay — your C application code stays identical.

Complete Picture — Everything Connected

DTS / Overlay ───────────────────────────────────────────────────── &i2c1 { pinctrl-0 = <&i2c1_scl_pb6 &i2c1_sda_pb7>; ← phandle-array (pinctrl) status = "okay"; mpu6050@68 { compatible = "invensense,mpu6050"; ← driver binding reg = <0x68>; ← I2C address interrupt-parent = <&gpioa>; ← simple phandle interrupts = <0 GPIO_ACTIVE_HIGH>; ← interrupt specifier int-gpios = <&gpioa 0 GPIO_ACTIVE_HIGH>; ← phandle-array (gpio) status = "okay"; }; }; ↓ DTC compiles everything ↓ devicetree_generated.h (in build/zephyr/include/generated/) ↓ Zephyr DT macros read it ↓ C Application ───────────────────────────────────────────────────── const struct gpio_dt_spec mpu_int = GPIO_DT_SPEC_GET(DT_NODELABEL(mpu6050), int_gpios); /* no hardcoded pin numbers, no hardcoded addresses */

Phandle Types Summary

| Type | Syntax | Example | Use Case |
| --- | --- | --- | --- |
| Simple phandle | = <&label> | i2c = <&i2c1> | One node references another |
| Phandle-array | = <&label cell1 cell2> | gpios = <&gpioa 5 1> | Reference + specifier data |
| Multiple phandles | = <&a &b> | pinctrl-0 = <&scl &sda> | Multiple references in one property |
| Interrupt parent | = <&label> | interrupt-parent = <&gpioa> | Which controller handles this interrupt |