ZEPHYR RTOS

Embedded RTOS / zephyr RTOS • 03-0-2026 • Aravinth K

zephy application on custom board

Device tree

What is device tree?

A data format and language for describing hardware in a platform-independent way. Device Tree describes what hardware exists, how it is wired, and how it is configured — so firmware and operating systems can set up devices without hardcoding board-specific details into application code.

Why Device Tree Matters

- In bare-metal coding for STM32, you write register-specific code directly in your application — hardcoded addresses, pin numbers, peripheral configs. This code cannot be reused on any other microcontroller (for example LPC or NRF52) because the registers and addresses are completely different.

- Device Tree solves this by separating board-specific hardware description from application logic:

Without Device Tree: application code contains hardcoded pin numbers, register addresses, peripheral configs → changing the board = rewriting the application With Device Tree: board hardware is described in .dts files application code asks "give me the LED device" → changing the board = only changing the .dts file application code stays exactly the same

Where to Find the DTS File

For the STM32F746G-DISCO board, the board DTS file is located at:

zephyr/boards/st/stm32f746g_disco/stm32f746g_disco.dts

DTS File Structure

Root Node

Everything in a Device Tree lives inside the root node /:

/ { /* all hardware description lives here */ };

The / is the root node — the top of the tree. All other nodes are children or descendants of it

Parent and Child Nodes

Device Tree is organized as a tree of nodes — exactly like a folder structure on your PC:

/ { ← root node leds { ← parent node (container) compatible = "gpio-leds"; green_led_1: led_1 { ← child node gpios = <&gpioi 1 GPIO_ACTIVE_HIGH>; label = "User LD1"; }; }; };

root / └── leds ← parent / container node └── led_1 ← child node

- leds is the parent node — a logical container that groups related hardware

- green_led_1 is the child node — describes one specific LED

- One parent can have many child nodes (multiple LEDs, multiple UARTs, etc.)

Properties

Inside every node, you define properties — key-value pairs that describe the hardware characteristics:

green_led_1: led_1 { gpios = <&gpioi 1 GPIO_ACTIVE_HIGH>; ← property label = "User LD1"; ← property };

| Property | Purpose |
| --- | --- |
| compatible | Tells Zephyr which driver to use for this node |
| gpios | Which GPIO port, pin number, and active state |
| label | Human readable name for the device |
| status | Whether the device is active ("okay") or disabled ("disabled") |
| reg | Memory address or device bus address |

Node Labels

Notice green_led_1: before led_1 — this is a node label:

green_led_1: led_1 { ← "green_led_1" is the label, "led_1" is the node name ... };

Labels let you reference this node from elsewhere in the DTS using &:

aliases { led0 = &green_led_1; ← referencing the node by its label };

Labels are like variable names for nodes. Without a label, you cannot reference a node from another part of the DTS.

Aliases Node

The aliases node is a special node at the root level that gives short, standardized names to hardware:

/ { aliases { led0 = &green_led_1; ← "led0" is the standard name apps use sw0 = &user_button; }; };

This is why the blinky sample works on any board — it always looks for led0 alias. The alias points to whatever the actual LED node is on that specific board. Your application code never needs to know the real node name.

Reopening a Node with &

You can reopen an already defined node anywhere in the DTS to modify or extend it:

&usart1 { pinctrl-0 = <&usart1_tx_pa9 &usart1_rx_pb7>; pinctrl-names = "default"; current-speed = <115200>; status = "okay"; };

- &usart1 — reopen the node that was already defined in an included .dtsi file

- status = "okay" — activate this peripheral (disabled by default in most SoC files)

- current-speed = <115200> — configure baud rate

- pinctrl-0 — assign physical pins to this peripheral

Most peripherals are defined in SoC-level .dtsi files with status = "disabled". You reopen them in your board .dts or app .overlay and set status = "okay" to activate them. This is the standard pattern across all Zephyr boards.

The compatible Property — Most Important Property

The compatible property is the most critical property in any node:

leds { compatible = "gpio-leds"; ← this string selects the driver };

Zephyr uses this string to match the node to its driver in the kernel:

DTS node compatible = "gpio-leds" ↓ Zephyr searches all registered drivers ↓ Finds driver that declares DT_COMPAT_DEFINE(gpio_leds, "gpio-leds") ↓ That driver is bound to this node automatically

Without compatible, Zephyr doesn't know which driver to load for the hardware. It is the bridge between hardware description and software driver.

DTS File Hierarchy — How Includes Work

A board's final DTS is not one file — it is assembled from multiple files included in a hierarchy:

/* stm32f746g_disco.dts */ #include <st/f7/stm32f746Xg.dtsi> ← SoC level (all STM32F746 chips) #include <st/f7/stm32f746nghx-pinctrl.dtsi> ← Pin definitions for this package #include "arduino_r3_connector.dtsi" ← Arduino header pin mapping #include <zephyr/dt-bindings/input/input-event-codes.h> #include <zephyr/dt-bindings/memory-attr/memory-attr.h> #include <zephyr/dt-bindings/memory-attr/memory-attr-arm.h>

The hierarchy from bottom to top:

ARM architecture base (arm.dtsi) ↓ SoC family level (stm32f7.dtsi) ← all STM32F7 chips ↓ SoC specific level (stm32f746Xg.dtsi) ← this specific STM32F746 chip ↓ Board level (stm32f746g_disco.dts) ← this specific board ↓ App overlay (stm32f746g_disco.overlay) ← your app's customizations ↓ Final merged zephyr.dts ← generated in build/ folder

Each layer adds or overrides what the layer below it defined. Your .overlay file at the top has the final say. This is why you can customize hardware behavior per-application without modifying any Zephyr source files.

.dtsi vs .dts vs .overlay

| Extension | Full Form | Purpose |
| --- | --- | --- |
| .dtsi | Device Tree Source Include | Reusable fragments (SoC, architecture level) |
| .dts | Device Tree Source | Complete board description |
| .overlay | Overlay | App-level customization on top of board .dts |

How Device Tree Reaches Your C Code

The DTS files are never directly used by your C code. They go through a compilation step:

.dts + .overlay files ↓ DTC (Device Tree Compiler) ↓ zephyr.dts (final merged file in build/) ↓ devicetree_generated.h (C header, also in build/) ↓ Your C code uses Zephyr DT macros to access it: const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(DT_ALIAS(led0), gpios);

Your C code never reads .dts files directly. The build system compiles the entire Device Tree into a generated C header, and you access it through Zephyr's DT API macros.